# Assessment Requirements for AHCARB209 Assist with aerial rescue from the ground

Release: 1

Assessment Requirements for AHCARB209 Assist with aerial rescue from the ground

# Modification History

| Release | TP Version | Comment |
| --- | --- | --- |
| 1 | AHCv1.0 | Initial release |

# Performance Evidence

The candidate must be observed assisting in a rescue from an EWP and participating in the rescue of a climber from the ground. The candidate must be assessed on their ability to integrate and apply the performance requirements of this unit in a workplace setting. Performance must be demonstrated consistently over time and in a suitable range of contexts.

The candidate must provide evidence for and demonstrate:

identifying site hazards, assessing risks and reporting to supervisor

confirming emergency communication system is working

confirming emergency services contact numbers are documented

selecting and checking suitable safety equipment, rescue kit and personal protective equipment

positioning rescue and first aid equipment for easy access

clarifying own role in rescue situations

confirm first aid, rescue personnel, equipment and rescue plan procedures with work team prior to undertaking work

selecting and using personal protective equipment

communicating with work team during operations using voice, hand and whistle signals

attempting communication with casualty

determining condition of casualty as accurately as possible

calling emergency services and electricity supply authority as required before assisting with rescue

determining if the EWP is in contact or closer than ‘safe approach distances’ with electrical wires and apparatus

determining EWP is structurally stable

maintaining ‘safe approach distances’

accessing EWP only if safe to do so

accessing truck-based controls and lowering bucket to ground if safe to do so

determining if tree or climber is in contact or closer than ‘safe approach distances’ with electrical wires and apparatus

maintaining communications with rescuer and casualty, work team and emergency personnel

keeping all ropes clear and free

observing rescue route and provide feedback to rescuer

assisting with managing casualty as they approach ground

determining if tree or climber is in contact or closer than ‘safe approach distances’ with electrical wires and apparatus

informing climber of presence of electrical wires

providing rescue equipment requested by the climber

installing or use existing high point as instructed

informing climber of lowering procedure

installing or using existing friction device to raise and lower the climber as required to lowering the climber to the ground

applying necessary first aid once casualty is on ground until medical professionals arrive

securing worksite and control entry until appropriate authorities inspect and release site

follow procedures for reporting accidents

isolating equipment involved in accident for potential investigation and preventing re-use until checked

removing all rescue equipment in a controlled manner

cleaning and storing rescue equipment

cleaning and storing personal protective equipment

maintaining records of aerial rescue in appropriate format

use of industry-standard terminology.

# Knowledge Evidence

The candidate must demonstrate knowledge of:

emergency procedures to assist in ground-based rescues of EWP and climbing workers

hazard identification methods

risk assessment systems

Safe Approach Distances

recognition of live electrical wires or apparatus

electrical wires and apparatus safety

first aid procedures

contents of a first aid kit

use of a first aid kit

emergency services communication systems

emergency services contact numbers

emergency plans

appropriate placement of rescue equipment

rescue kits

rescue personnel, equipment and rescue plan procedures

how to control risk to rescuer, victim and others

types of reporting required after rescue

personal protective equipment

how to deal with an emergency

lowering mechanisms for an aerial rescue

work health and safety records with regards to incident reporting.

# Assessment Conditions

Assessment must be demonstrated consistently over time in a suitable range of contexts and have a productivity-based outcome. No single assessment event or report is sufficient to achieve competency in this unit.

Assessment may be conducted in a simulated or real work environment, however determination of competency requires the application of work practices under work conditions.

The mandatory equipment and materials used to gather evidence for assessment include:

equipment:

rescue kit

rescue dummy

high decibel whistle

rigging equipment

static and dynamic rope kit

harness

lowering and friction devices

personal protective equipment (PPE)

first aid and emergency kit

traffic management kit

signage – work zone

trees

materials:

hazard identification and risk control form

equipment and PPE check form

emergency preparation form

rescue assistance form

work communications form

Assessors must satisfy current standards for RTOs in the assessment of arboriculture units of competency.

Assessment must be conducted only by persons who have:

arboriculture vocational competencies at least to the level being assessed

current arboriculture industry skills directly relevant to the unit of competency being assessed

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=c6399549-9c62-4a5e-bf1a-524b2322cf72