# AHCIRG220 Assist with surface irrigation operations

Release: 1

AHCIRG220 Assist with surface irrigation operations

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with AHC Agriculture, Horticulture and Conservation and Land Management Training Package Version 4.0. |

# Application

This unit of competency describes the skills and knowledge required to assist with installing, operating and maintaining surface irrigation systems.

The unit applies to individuals who assist with surface irrigation operations under general supervision with limited autonomy and accountability.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Irrigation (IRG)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Plan and prepare for work | 1.1 Read work instructions and confirm with supervisor activity to be undertaken, including identifying potential hazards and risks and implementing safe working practices to minimise risks to self and others 1.2 Select appropriate tools and equipment for the task 1.3 Select, fit and use personal protective equipment and clothing applicable to the task 1.4 Identify site for installation of the irrigation system |
| 2. Set up field for surface irrigation systems as directed | 2.1 Check irrigation gates, channels and ditch-ends 2.2 Check pumps, bores and other water delivery mechanisms for correct irrigation set up 2.3 Position and secure tarpaulins or other water control devices as directed |
| 3. Carry out routine maintenance activities on irrigation delivery systems as directed | 3.1 Assist with mechanical equipment servicing according to supervisor instructions 3.2 Flush and clean supply and distribution system as directed 3.3 Maintain system inlets, outlets, structures and fittings 3.4 Check system is running smoothly and is free of damage, leaks and blockages in channels, drains and outlets 3.5 Clean irrigation system and ensure there is no disruption to the system |
| 4. Undertake surface irrigation work as directed | 4.1 Open and shut gates and valves 4.2 Ensure required head and water levels in head ditch are achieved and maintained to ensure sufficient water flow and availability to crops 4.3 Start or open the required number of siphons 4.4 Monitor progress of water flow in furrows 4.5 Lift siphons where irrigation is complete 4.6 Carry out irrigation change and mark 4.7 Use mechanical equipment to build and repair banks and for weed removal 4.8 Move irrigation equipment according to supervisor instructions |
| 5. Record and report maintenance activities | 5.1 Record damage and blockage caused by weeds by damage type, location and the section of the system affected 5.2 Record and report damaged or faulty pumps, valves, electrical components and computer systems 5.3 Record and report routine maintenance activities |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interpret textual information from a range of sources to identify relevant and key information about workplace operations |
| Oral communication | Use clear language to confirm work activity and report irrigation system damage, faulty equipment, blockages and routine maintenance activity |
| Numeracy skills | Identify irrigation system component part numbers |
| Navigate the world of work | Recognise and follow workplace requirements, including safety requirements, associated with own role and area of responsibility |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| AHCIRG220 Assist with surface irrigation operations | AHCIRG216 Assist with surface irrigation operations | Performance criteria clarified Foundation skills added Assessment requirements updated | Equivalent unit |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=c6399549-9c62-4a5e-bf1a-524b2322cf72