# Assessment Requirements for AHCLPW307 Perform restricted diving for scientific purposes

Release: 1

Assessment Requirements for AHCLPW307 Perform restricted diving for scientific purposes

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with AHC Agriculture, Horticulture and Conservation and Land Management Training Package Version 6.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has prepared for scientific dives and has:

interpreted dive objectives and planned the equipment, resources, and consumables required to carry out a dive

prepared resources, equipment and dive team for dive: including:

inspected equipment for signs of deterioration, damage or corrosion and tested for proper operation

selected equipment for dive and ensured correct storage during transport

used decompression tables, including Defence and Civil Institute of Environmental Medicine tables (DCIEM) to calculate safe diving profiles, including:

depth increments

time increments

stop time

ascent rates

total decompression time

bottom time

total dive time

surface interval

used decompression tables to calculate the following without error:

‘No stop’ bottom time for a given depth

decompression depth/time increments

repetitive dive time increments

prepared compressed air and enriched air nitrox cylinders (EAN) for diving operations according to legislative requirements, including:

inspected and maintained compressors

determined cylinder pressure for dive

charged air cylinders using high pressure air compressor according legislative requirements, and workplace health and safety procedures

charged air cylinders by decanting according legislative requirements, and workplace health and safety procedures

tested the quality of gas

analysed oxygen content of dive mix for EAN

used approved cylinder types for compressed air and EAN according to Industry standards and workplace health and safety procedures

maintained positive pressure in EAN cylinders to prevent ingress of atmospheric air

carried out pre-dive checks and performed a safety risk assessments for dive site

ensured availability of first aid equipment, and oxygen administration kit

monitored dive conditions, responded to incidents and amended dive procedures according to identified issues.

There must also be evidence that the individual has undertaken dives using compressed air and EAN in open water to a maximum depth of 30 metres which must include the following dives:

one dive between 27 metres and 30 metres

two dives with zero visibility in two to nine metres depth of water with a minimum of 20 minutes per dive

one night dive

four boat dives

four dives at depths greater than 20 metres, simulating decompression stops during ascent

at least three dives must be undertaken using EAN procedures including:

selected the blend and equipment to meet industry and safety criteria for EAN operations

conducted dive preparation for EAN dives according to industry standards, workplace health and safety policies and equipment manufacturer operating procedures

completed EAN dive according to dive plan and EAN procedures

Dives must be undertaken in conformance with statutory work health and safety, Australian Standards and codes of practices, with evidence the individual has:

fitted and removed dive equipment and assisted other divers to fit and remove dive equipment in correct sequence

selected water entry and exit methods appropriate to the situation and water conditions

entered and exited water safely with no damage to equipment and assisted other divers to enter and exit water

checked and reported dive equipment for leaks on entering water

conducted dive in open water safety according to dive plan, including:

adapted operational techniques according to prevailing conditions

used buoyancy control techniques and equipment appropriate to diving conditions

demonstrated correct breathing techniques

demonstrated decent and ascent techniques appropriate to the dive conditions

demonstrated safety-critical underwater skills during dive, including:

monitored gas consumption and gas supply and ascended when predetermined level of gas remained to prevent exhaustion of gas supply

changed over to reserve gas supply and notified surface

managed life line to minimise entanglement and facilitate line signals

followed life line to surface

monitored depth and time throughout the dive

conducted dive practices for compressed air and EAN within diving maximum limits to meet industry technical standards and workplace health and safety policies and procedures

conducted specified dive operations according to objectives of operational dive plan

used basic non-powered hand tools to complete objectives according to planned dive operations

checked, cleaned maintained and stored hand tools according to workplace procedures

communicated with others in the dive team underwater, diver to diver and diver to surface using each of the following:

hand signals

line signals

voice communications

demonstrated decompression procedures according to dive profile using each of the following:

lazy shot rope

fixed shot rope

lazy shot rope during drift dives

lazy shot using a life line

float line

accent rate control and depth control at water stops

performed at least two of the following search techniques during the dive:

circular

jackstay grid

snag line

grid mesh

semi-circular

parallel

performed at least two of the following survey techniques during the dive:

line transect

quadrat search

photographic quadrat

video transect

timed swim

measured time swim

used basic rope work and rigging techniques during dive operations using the following knots:

reef knot

bowline

clove hitch

rolling hitch

round turn and two half hitches

sheet bend

figure of eight

figure of eight on the bite

established effective working relationships with dive team, including:

established communications under water

monitored dive team members for compliance with ‘no-decompression’ requirements

debriefed dive team after dive activity

communicated and activated an emergency management plan and taken action in an emergency situation including:

used emergency drills, procedures and methods

acted as a surface standby diver

acted as an in-water standby diver when divers are near enough to communicate and act as standby diver for each other

performed an effective in-water rescue of an unconscious diver to boat or shore

acted in a self-rescue emergency situation

performed first aid on diving casualty which must include:

recognised and assessed diver for adverse health conditions

determined if emergency services are required and obtain and provide support

administer first aid to diving casualty promptly according to workplace procedures

used an oxygen administration kit

performed post dive personnel and equipment checks, including:

check and report supporting diver condition

read and record cylinder pressure

reviewed dive plans following debriefing and risk analysis

disassembled, cleaned, inspected, maintained and stored compressed air and EAN diving equipment according to manufacturer instructions including:

ensured EAN compatible parts and processes for elevated oxygen mixture according to industry standards and workplace health and safety procedures

reported equipment faults according to workplace procedures following dive

maintained records according to industry standards and workplace requirements.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

industries associated with underwater work and the type of work performed

qualifications, legislative and regulatory requirements for occupational divers including restricted occupational SCUBA diving and occupational health and safety legislation as it relates to divers and their employers.

dive planning processes, including risk assessment, hazard identification and control

additional planning required for EAN diving, including:

requirements of EAN use and associated hazards

aspects of dive plan where EAN specific considerations are needed

use of industry recognised decompression tables and dive computers to plan and carry out safe diving practices

anatomy, physiology and physics of underwater diving, including:

anatomical and physiological systems of the human body including respiratory, circulatory and nervous systems, musculoskeletal, cardiovascular, ears, sinuses, vestibular organs, gastrointestinal, endocrine and immune systems

common units of measurement and the relationship between pressure, volume and temperature in the context of diving

definitions of atmospheric pressure, hydrostatic pressure, absolute pressure, ambient pressure and gauge pressure

Boyles Law and relationship between pressure and volume and the calculations relevant to depth and pressure

barotrauma and typical barotrauma injuries

use of decompression tables to determine safe diving profiles

partial pressure and gas solubility and their effect on the human body including Daltons Law

basic relationship between pressure and temperature including Charles Law, and calculating pressure changes with temperature

effect of buoyancy and effect on divers and underwater objects including Archimedes Principle including, diffusion, saturation, desaturation, equilibrium

behaviour of light and sound underwater and impact on divers, including equipment used to compensate

impact of temperature on cylinder pressure

heat loss and impact on diver

decompression as it relates to partial pressure and solubility of gasses (Henry’s Law)

inert gas narcosis

diving equipment, testing, operation and maintenance including:

selecting diving equipment for operational activities

assembly and disassembly of equipment used in dive operations

pre and post dive checks and maintenance

selection of appropriate diving equipment for required tasks such as masks, fins, cylinders, regulators, buoyancy compensation devices, harnesses, weights and safety devices

compressors for air and EAN cylinder charging

safety procedures

inspecting equipment and signs of disrepair

gas quality and gas testing standards used in Australia

record keeping practices for equipment

correct fitting and removal of dive equipment on divers including assisting others on fitting and removal

types of ropes used in diving operations their purpose, application, including:

care and maintenance

basic rigging techniques

knots and other rigging devices used in diving operations their purpose and application

tools and equipment for scientific diving operations, their use and maintenance

navigation, search and recovery techniques and their application in scientific diving

survey techniques and their application in scientific diving

health and safety risks associated with diving, including:

statutory health and safety requirements

standards and codes of practice

roles and responsibilities

risk assessment and developing control measures

emergency procedures and their implementation, including:

dive rescue procedures

oxygen administration procedures

agency standing orders for notification

emergency service procedures

recognising the need for decompression chamber and support

first aid for divers, including:

recognising signs and symptoms of diving related ill health including, decompression illness, mask and suit squeeze, ear and sinus problems, near drowning, vomiting under water, gas embolism and pulmonary barotrauma, breathing gas contamination, carbon monoxide poisoning, carbon dioxide poisoning, oxygen toxicity, anoxia and hypoxia, hypothermia and hyperthermia, injury/illness from marine organisms

assessing when first aid is required

administering first aid to divers

use of defibrillators in a wet environment

compression chamber theory

specialised diving modes including but not limited to zero and low visibility diving, tethered, night and deep diving.

effective teamwork and team communications, including

underwater communication procedures and equipment

statutory and workplace record keeping procedures.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

skills must be demonstrated in open water to a depth of no greater than 30 metres

conditions must comply with the current version of the Australian Standard for training and certification of occupational divers - restricted occupational SCUBA diver

resources, equipment and materials:

SCUBA equipment

safety equipment

compressors for filling cylinders with air and enriched air nitrox

cylinders for filling and use

tools and equipment for conducting dives, searches and surveys

specifications:

workplace documents such as policies, procedures, processes, forms

manufacturer’s operating instructions for equipment

workplace instructions and job specifications including risk assessment requirements

legislation, codes of practice and standards for occupational diving

relationships:

dive team member

timeframes:

dives must comply with times specified in no-decompression tables and calculations.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

In addition, the following specific assessor requirements apply to this unit:

assessors must be qualified as occupational SCUBA diving instructor through a recognised diver training organisation.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=c6399549-9c62-4a5e-bf1a-524b2322cf72