# Assessment Requirements for AMPREN301 Operate and monitor a batch rendering process

Release: 1

Assessment Requirements for AMPREN301 Operate and monitor a batch rendering process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with AMP Australian Meat Processing Training Package Version 7.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has set up, operated, monitored and shut down industrial batch rendering processes to convert raw materials into finished products according to specific work orders on at least two occasions.

For each process, there must be evidence that the individual has:

confirmed the product quantity and quality requirements

identified at least three food safety hazards, assessed and controlled risk using Hazard Analysis and Critical Control Point (HACCP) principles

identified at least three safety hazards, assessed and controlled risk using workplace health and safety procedures

selected and correctly fitted appropriate personal protective equipment (PPE) and clothing required for worker safety and food contamination control

checked raw materials, including correct labels, temperature, moisture, particle size and colour, set up and calibrated rendering equipment, including temperature, and rendering duration

loaded and unloaded raw materials, and monitored and adjusted their flow and quality according to product requirements

observed temperature and pressure gauges and adjusted equipment to meet product and safety requirements

collected and tested at least three product samples to ensure rendered specifications are met

identified and taken corrective action for two of the following:

over-rendering

under-rendering

spills

cross contamination

applied liquid discharge management requirements

monitored environmental emissions according to workplace and Environmental Protection Agency (EPA) requirements, including odour control and one of the following:

waste water

heat recovery

noise

communicated verbally on batch rendering process, product or equipment using industry terminology with a supervisor, and at least one of the following:

maintenance team member

quality assurance team member

environmental management team member

completed production records, including product and equipment issues, according to workplace requirements.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

basic operating principles of the batch rendering process

features and functions of batch rendering machinery and equipment, including:

manufacturer and workplace instructions for safe operation

control panels and ancillary controls

operating capacities, efficiencies and applications

location and purpose of guards, rails and sensors

feedback instrumentation

services required for operating a batch rendering process, including:

compressed air

electricity and water

workplace health and safety requirements

key features of HACCP techniques related to batch rendering processes

workplace procedures for safe movement of workers and visitors in and around the plant, rendering machinery and equipment

features, benefits, limitations, and correct fit of PPE for batch rendering processes

raw materials quality requirements, including:

temperature

particle size

cooked meat product requirements, including:

moisture content

structure, including friability, density

key features of quality assurance aspects of cooked product, including:

colour and density of cooked products

end-point temperature

correct level of moisture content

permissible bacteria types and quantity/quality requirements of finished products including:

tallow moisture, insolubles and Free Fatty Acids (FFA)

meat meal moisture, fat content, protein, microbiological and physical contamination

implications on product quality and productivity of incorrect load in cooker

effects of over-rendering and causes of typical faults and how they can be rectified according to workplace requirements, including:

bleached colour of the tallow

effect of fines on tallow quality

damage to amino acids

AS5008 Hygienic Rendering of Animal Products

Australian Renderers Association’s Code of Practice for Hygienic Rendering of Animal Products

workplace cleaning and maintenance requirements for batch cooking plant and equipment

workplace and EPA requirements for monitoring environmental emissions, including odour control, waste water, heat recovery and noise as applicable.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

skills must be demonstrated in a meat rendering plant or an environment that accurately represents workplace conditions

resources, equipment and materials:

industrial batch cooking plant and equipment required for the tasks specified in the performance evidence

raw materials specified in the performance evidence

PPE and contamination control equipment and clothing required for the tasks specified in the performance evidence

specifications:

two batch cooking work orders stipulated in the performance evidence

workplace procedures for a batch cooking process and operation of plant and equipment stipulated in the performance evidence

workplace procedures for worker safety and contamination control

relationships:

supervisor and others stipulated in the performance evidence

timeframes:

competency must be demonstrated at the normal rate of production for the rendering plant.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=5e2e56b7-698f-4822-84bb-25adbb8443a7