# AMPREN305 Operate a blood coagulation and drying process

Release: 1

AMPREN305 Operate a blood coagulation and drying process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with AMP Australian Meat Processing Training Package Version 7.0. |

# Application

This unit of competency describes the skills and knowledge required by workers to operate a blood coagulation and drying process including preparation, loading raw material, operation, monitoring and shut down.

This unit applies to individuals who work under broad direction in an industrial rendering plant or abattoir and who are responsible for operating and monitoring a blood coagulation and drying process. It includes safety compliance using Hazard Analysis and Critical Control Point (HACCP) techniques, process and product quality requirements, and solving problems that arise with the process.

All work must be carried out to comply with workplace procedures, according to state/territory health and safety, environmental and meat rendering regulations, legislation and standards that apply to the workplace.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Rendering (REN)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for blood coagulation and drying process | 1.1 Check work order, operational status of services and availability of consumables 1.2 Identify hazards, assess and control risks according to workplace health and safety procedures 1.3 Identify food safety hazards, assess and control risks according to HACCP principles 1.4 Select and fit personal protective equipment and contamination control equipment 1.5 Undertake preoperational checks of machinery and equipment according to workplace procedures and/or manufacturer requirements 1.6 Transfer blood from slaughter floor or transport vehicles, into holding tanks and add appropriate chemicals. 1.7 Visually monitor the quantity and quality of raw blood product to ensure required specifications are met |
| 2. Undertake blood coagulation and separation | 2.1 Start blood coagulation and separation process 2.2 Control steam flow and raw blood flow to facilitate coagulation of blood 2.3 Monitor coagulation of blood and separation according to process specifications or process requirements 2.4. Conduct visual checks to determine colour of stick water discharge 2.5 Use modulating valve to control temperature for blood coagulation 2.6 Identify and correct out-of-specification processes and product |
| 3. Operate blood drying process | 3.1 Regulate flow of coagulated blood into dryer and monitor dryer discharge temperature 3.2 Operate equipment and monitor blood meal for moisture content 3.3 Take blood meal samples to determine moisture content |
| 4. Operate screening and milling of blood meal | 4.1 Check milling and vibrating screens for holes and blockages to ensure correct particle size in blood meal 4.2 Check conveying equipment to ensure it is dry and clean to avoid bacterial contamination 4.3 Monitor odours and other environmental emissions according to workplace procedures 4.4 Operate screening and milling equipment and monitor process 4.5 Take blood meal samples for salmonella testing according to quality assurance (QA) program 4.6 Identify and rectify processing faults according to workplace procedures |
| 5. Package blood meal and load out | 5.1 Bag blood meal and apply labels according to product specifications, quality control and regulatory requirements 5.2 Store packaged blood meal in a clean dry environment and isolate product for holding and testing 5.3 Isolate samples of bagged product for future testing according to QA program 5.4 Complete load out documentation as per QA program and dispatch for transport |
| 6. Shut down coagulation and drying process | 6.1 Clean blood coagulation and drying equipment with water and approved chemicals according to QA program 6.2 Shut down the equipment safely according to operating procedures 6.3 Identify and report maintenance requirements according to workplace procedures 6.4 Maintain workplace records in appropriate format 6.5 Conduct handover of blood coagulation and drying operation to another team member when required |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interpret operating manuals for blood processing machinery and equipment Read and interpret gauges and symbols |
| Writing | Complete records using industry terminology |
| Oral Communication | Use open and closed questions to clarify information |
| Numeracy | Estimate and measure volume (mls, L), temperature (degrees Celsius), moisture percentage, steam flow and blood flow |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| AMPAREN305 Operate a blood coagulation and drying process | AMPA3088 Operate blood processing plant | Merged with AMPA3089 Produce rendered products hygienically Unit title and application changed to better reflect work task Unit sector added, unit code updated Previous element 1 removed and new element 1 added New elements 2,3,4,5,6 added Performance criteria added, revised, reordered, or merged for clarity Foundation skills updated Performance Evidence, Knowledge Evidence and Assessment Conditions updated | Not equivalent |
| AMPAREN305 Operate a blood coagulation and drying process | AMPA3089 Produce rendered products hygienically | Merged with AMPA3088 Operate blood processing plant Unit title and application changed to better reflect work task Unit sector added, unit code updated | Not equivalent |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=5e2e56b7-698f-4822-84bb-25adbb8443a7