# Assessment Requirements for AURETH001 Depower and reinitialise battery electric vehicles

Release: 1

Assessment Requirements for AURETH001 Depower and reinitialise battery electric vehicles

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standard defined in the unit’s elements, performance criteria, range of conditions and foundation skills:

safely depower the rechargeable energy storage system (RESS) and separated extra low voltage (SELV) system of two different battery electric vehicles (BEVs), and stabilise the vehicles ensuring that no residual high voltage (HV) is measured

safely reinitialise each BEV’s RESS and SELV system and confirm correct vehicle operation.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to depowering and reinitialising BEVs, including procedures for:

identifying hazards and controlling risks associated with:

working with high voltages on BEV electrical systems

wearing jewellery while working around high electrical currents

determining appropriate procedures for minimising risk associated with hazards, including applying electrical safety precautions when:

using personal protective equipment (PPE), including electrical safety gloves with 1000 volt rating and Australian standards rated HV insulating mat

identifying and using firefighting equipment as appropriate

using the ‘one hand’ rule

following live system warning tags and signs

depowering vehicle

isolating HV RESS electrical supply

stabilising vehicle electrical system.

requirements of AS 5732 Electric vehicle operations - Maintenance and repair

operating principles of RESS and SELV electrical systems relevant to a range of BEVs

application, purpose and operation of a range of vehicle RESS depowering and reinitialising methods.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the BEVs that they have depowered and reinitialised e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

PPE and safety equipment, including electrical safety gloves with 1000 volt rating and Australian standards rated HV insulating mat

manufacturer specifications and service procedures for the BEVs being depowered and reinitialised

AS 5732 Electric vehicle operations - Maintenance and repair

two different BEVs as specified in the performance evidence

electrical test equipment, including:

digital multimeter with Cat III 1000 volt rating

insulation tester

residual voltage tester, if specified in original equipment manufacturer (OEM) test requirements

scan tool

tools, equipment and materials appropriate for depowering and reinitialising BEVs.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1