# Assessment Requirements for AURETH012 Service and maintain electrical components in hybrid electric vehicles

Release: 1

Assessment Requirements for AURETH012 Service and maintain electrical components in hybrid electric vehicles

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standard defined in the unit’s elements, performance criteria, range of conditions and foundation skills:

safely test, service and maintain two different hybrid electric vehicles (HEVs) or plug-in hybrid electric vehicles (PHEVs).

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to servicing and maintaining HEV and PHEV electrical components, including procedures for:

identifying hazards and controlling risks associated with:

working with high voltages on HEV and PHEV electrical systems

wearing jewellery while working around high electrical currents

determining appropriate procedures for minimising risk associated with hazards, including applying electrical safety precautions when:

using personal protective equipment (PPE), including electrical safety gloves with 1000 volt rating and Australian standards rated high voltage (HV) insulating mat

identifying and using firefighting equipment as appropriate

using the ‘one hand’ rule

following live system warning tags and signs

depowering vehicle

isolating HV rechargeable energy storage system (RESS) electrical supply

stabilising vehicle electrical system

requirements of AS 5732 Electric vehicle operations - Maintenance and repair

key electrical components of HEVs and PHEVs and their functions

RESS battery construction, including:

battery internal resistance

battery types

terminal corrosion

terminal resistance

key components of RESS theory, including:

cell failure theory

charging characteristics

open circuit cells

reverse polarisation

series cell configuration

strapping and layout

different types of HEVs and PHEVs, including:

series, parallel and series parallel

strong, mid or weak hybrid systems

operating principles of RESS and battery management system (BMS)

key characteristics of HV battery charger and direct current (DC) to DC converter

operating principles of power distribution unit (PDU).

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the electrical systems and components they have serviced and maintained in HEVs or PHEVs, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

PPE, including electrical safety gloves with 1000 volt rating and Australian standards rated HV insulating mat

manufacturer specifications and service procedures for the HEVs or PHEVs being worked on

AS 5732 Electric vehicle operations - Maintenance and repair

two different operational HEVs or PHEVs requiring service and maintenance

electrical test equipment appropriate to the HEVs and PHEVs being serviced, including:

digital multimeter with Cat III 1000 volt rating

insulation tester

residual voltage tester, if specified in original equipment manufacturer (OEM) test requirements

scan tool

tools, equipment and materials appropriate for servicing and maintaining HEVs and PHEVs.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1