# Assessment Requirements for AURETH109 Diagnose and repair DC to DC converters in battery electric vehicles

Release: 1

Assessment Requirements for AURETH109 Diagnose and repair DC to DC converters in battery electric vehicles

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with AUR Automotive Retail, Service and Repair Training Package Version 6.0 |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

diagnose and repair the DC to DC converter and its system components on at least two different battery electric vehicles (BEVs) to correct performance deficiencies.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

methods to locate and interpret information required to diagnose and repair DC to DC converters in battery electric vehicles, including:

information provided by customers and supervisors

Australian Standards (AS) 5732 Electric vehicle operations - Maintenance and repair

manufacturer specifications and procedures

workplace procedures required to diagnose and repair DC to DC converters in battery electric vehicles, including:

establishing serviceability of tools and equipment

documentation procedures

housekeeping procedures, including:

examination of tools and equipment

storage of equipment

identification, tagging and isolation of faulty equipment

disposal of excess materials

recycling procedures

workplace health and safety (WHS) requirements relating to diagnosing and repairing DC to DC converters in battery electric vehicles, including procedures for:

identifying hazards and controlling risks associated with:

working with high voltages in BEV electrical systems

wearing jewellery while working around high electrical currents

determining appropriate procedures for minimising risk associated with hazards, including applying electrical safety precautions when:

using personal protective equipment (PPE), including electrical safety gloves with 1000 volt rating and Australian standards rated HV insulating mat

identifying and using firefighting equipment

using the one hand rule

following live system warning tags and signs

depowering vehicle

isolating HV RESS electrical supply

stabilising vehicle electrical system

environmental requirements relating to diagnosing and repairing DC to DC converters in battery electric vehicles, including procedures for trapping, storing and disposing of waste produced during repair.

safe operating procedures for tools and equipment, including:

digital multimeter with Cat III 1000 volt rating

insulation tester

residual voltage tester

scan tool

oscilloscope

operating principles of DC to DC converters in battery electric vehicles, including:

step-down converters

step-up converters

continuous current mode

discontinuous current mode

signal noise, including radio frequency (RF) noise, input noise and output noise

purpose and operation of DC to DC converters in battery electric vehicles, including:

linear converters

switched mode conversion

magnetic converters

capacitive converters

diagnostic testing procedures for DC to DC converters in battery electric vehicles, including procedures for:

accessing and interpreting scan tool system data, including:

diagnostic trouble codes (DTCs)

live data

using diagnostic flow charts

electrical system testing, including procedures for:

accessing electrical terminals and using test probes without damaging connectors, fuse holders or wiring

checking voltage input and output of DC to DC converters

checking routing and damage to cabling

repair procedures for DC to DC converters in battery electric vehicles, including procedures for:

tightening connections

replacing faulty or damaged cable connections

removing and replacing faulty or damaged components

post-repair testing procedures for DC to DC converters in battery electric vehicles, including procedures for:

DTC clearing procedures

checking for electrical connector mating.

# Assessment Conditions

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the BEV DC to DC converters that they have worked on, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

PPE and safety equipment, including electrical safety gloves with 1000 volt rating and HV insulating mat with Australian standards rating

manufacturer specifications for BEV DC to DC converters

AS 5732 Electric vehicle operations: Maintenance and repair

two different BEVs with DC to DC converters accessible for diagnostic and repair or replacement activities

electrical diagnostic equipment appropriate to the BEV being diagnosed and repaired, including:

digital multimeter with Cat III 1000 volt rating

insulation tester

scan tool

oscilloscope

residual voltage tester, if specified in original equipment manufacturer (OEM) test requirements

tools, equipment and materials appropriate for repairing DC to DC converters in BEVs.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1