# AURETR026 Remove, replace and program electrical and electronic units and assemblies

Release: 1

AURETR026 Remove, replace and program electrical and electronic units and assemblies

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Application

This unit describes the performance outcomes required to remove, replace and program electrical and electronic units and assemblies integral to vehicle and machinery controller area network databus (CAN-bus). Electrical and electronic units and assemblies include powertrain control modules, engine and body control modules, and other electronic control modules that may require manufacturer programming procedures.

It applies to those working in the automotive service and repair industry. The electrical and electronic units and assemblies include those fitted to agricultural machinery, heavy commercial vehicles, light vehicles, vessels, mobile plant machinery, motorcycles or outdoor power equipment.

No licensing, legislative, regulatory or certification requirements apply to this unit at the time of publication.

# Competency Field

Electrical

# Unit Sector

Technical - Electrical and Electronic

# Elements and Performance Criteria

| Elements Elements describe the essential outcomes. | Performance Criteria Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold and italicised text is used, further information is detailed in the range of conditions section. |
| --- | --- |
| 1. Prepare to remove, replace and program electrical and electronic units and assemblies | 1.1 Job requirements are determined from workplace instructions 1.2 Workplace procedures and manufacturer specifications are accessed and interpreted 1.3 Removal information is accessed and interpreted from manufacturer and component supplier specifications 1.4 Hazards associated with the work are identified and risks are managed 1.5 Tools and equipment are selected and checked for serviceability |
| 2. Remove unit and assembly | 2.1 Unit and assembly are removed using approved methods, tools and equipment and according to manufacturer specifications, workplace procedures and safety requirements 2.2 Unit and assembly are handled and stored according to manufacturer and component supplier requirements |
| 3. Replace unit and assembly | 3.1 Replacement information is accessed and interpreted from manufacturer and component supplier specifications 3.2 Unit and assembly are replaced using approved methods, tools and equipment and according to manufacturer specifications and workplace procedures |
| 4. Program unit and assembly | 4.1 Reprogramming options are analysed and those most appropriate are selected 4.2 Component replacement and programming procedures are carried out according to manufacturer and component supplier specifications 4.3 Post-replacement testing is carried out to confirm that unit and assembly are operating to manufacturer specifications and that no other problems are present |
| 5. Complete work processes | 5.1 Final inspection is made to ensure work is to workplace expectations and vehicle or machinery is presented ready for use 5.2 Work area is cleaned, waste and non-recyclable materials are disposed of, and recyclable material is collected 5.3 Tools and equipment are checked and stored and any faulty electrical equipment is identified, tagged and isolated according to workplace procedures 5.4 Workplace documentation is processed according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance and are not explicit in the performance criteria.

| Skills | Description |
| --- | --- |
| Learning skills to: | locate appropriate sources of information efficiently. |
| Reading skills to: | interpret text, symbols and diagrams in manufacturer specifications, safe operating procedures and workplace information. |
| Writing skills to: | legibly and accurately fill out workplace documentation. |
| Oral communication skills to: | clarify instructions. |
| Numeracy skills to: | match component part numbers to workplace instructions and vehicle and component part lists interpret measurements of voltage, current and resistance relating to electrical and electronic units and assemblies measure components to determine compliance with specifications. |
| Planning and organising skills to: | plan own work requirements and prioritise actions to achieve required outcomes and ensure tasks are completed within workplace timeframes. |
| Problem solving skills to: | determine cause of faults and select most appropriate diagnostic and reprogramming method. |
| Technology skills to: | use specialised workplace technology, tools and equipment relating to removing, replacing and programming electrical and electronic units and assemblies, including scan tools. |

# Range of Conditions

This section specifies work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. Bold italicised wording, if used in the performance criteria, is detailed below.

| Safety requirements must include: | work health and safety (WHS) and occupational health and safety (OHS) requirements, including procedures for identifying hazards and controlling risks associated with wearing jewellery while working around high current wiring systems. |
| --- | --- |

# Unit Mapping Information

Equivalent to AURETR3026 Remove, replace and program electrical and electronic units and assemblies

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Assessment Requirements for AURETR026 Remove, replace and program electrical and electronic units and assemblies

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standard defined in the unit’s elements, performance criteria, range of conditions and foundation skills:

remove, replace, program and retest two of the following systems on controller area network databus (CAN-bus) circuits in different vehicles or machinery, including:

supplementary restraint system (SRS) module

body control module (BCM)

engine control module (ECM)

powertrain control module (PCM)

electric brake control (EBC), antilock braking system (ABS) or vehicle stability control (VSC) module

collision avoidance control (CAC) module

adaptive cruise control (ACC) module.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to removing, replacing and programming electrical and electronic units and assemblies, including procedures for identifying hazards and controlling risks associated with wearing jewellery while working around high current wiring systems

key features of electrical and electronic units and assemblies integral to vehicle CAN-bus networks, including:

supplementary restraint system modules

body control modules

engine control modules

powertrain control modules

electric brake control, antilock braking system, and vehicle stability control modules

collision avoidance control modules

adaptive cruise control modules

various types of wiring systems found in vehicles, including:

basic wiring

twisted pair

shielded wiring

CAN-bus wiring

procedures for removing and replacing electrical and electronic units and assemblies, including:

identifying unit position using wiring schematics and drawings

identifying unit model and configuration

identifying visual indications of unit damage, including:

physical damage and wear

water and moisture ingress

avoiding static electricity discharge when handling units

disconnecting and reconnecting electrical connectors

procedures for programming electrical or electronic units, including:

service programming system (SPS)

original equipment manufacturer (OEM) programming code

flash code or program

vehicle configuration index (VCI) code or password

post-replacement testing procedures, including checking unit system operation

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the electrical and electronic units and assemblies they have removed, replaced and programmed, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

manufacturer electrical and electronic unit and assembly specifications

two different vehicles or machinery requiring removal, replacement and programming of electrical and electronic units and assemblies

tools, equipment and materials appropriate for removing, replacing and programming electrical and electronic units and assemblies, including scan tool.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1