# Assessment Requirements for AURETR029 Diagnose and repair charging systems

Release: 1

Assessment Requirements for AURETR029 Diagnose and repair charging systems

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standard defined in the unit’s elements and performance criteria, range of conditions and foundation skills:

diagnose and repair a fault in two of the following charging system components in two different vehicles, vessels or machinery:

alternator stator or rotor circuit

alternator internal regulator control circuit

external regulator battery management system circuit

battery to alternator wiring and earthing system circuit

machinery charging system circuit.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to diagnosing and repairing charging systems, including procedures for:

using specialised tools and equipment

using appropriate personal protective equipment (PPE)

identifying hazards and controlling risks associated with:

working on high voltage ignition systems

wearing jewellery while working around high current wiring systems

operating principles of charging systems and associated components, including:

charging batteries with direct current

vehicle system power and electrical current requirements

generating principles, including Faraday’s law and inducing an electromotive force (EMF)

producing direct current (DC) EMF

producing alternating current (AC) EMF

changing AC to DC, including half-wave and full-wave rectification

application, purpose and operation of charging systems and components, including:

generators, including:

internal component function and operation

regulation of output voltage and current

alternators, including:

internal component function and operation

star-connected and delta-connected stator windings

regulation of output voltage, including zener diodes and exciter diodes

battery management systems

diagnostic testing procedures for charging systems, including:

accessing and interpreting scan tool system data, including:

diagnostic trouble codes (DTCs)

live data

freeze frame data

waveforms

using diagnostic flow charts

testing electrical systems, including procedures for:

accessing electrical terminals and using test probes without damaging connectors, fuse holders or wiring

load testing charging systems

resistance, current flow and voltage drop checks of charging system circuits

repair procedures for charging systems, including:

tightening connections

replacing faulty or damaged cable connections

removing and replacing faulty or damaged components

post-repair testing procedures for charging systems, including:

DTC clearing procedures

checking for electrical connector mating

static and dynamic performance tests of charging systems.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the charging systems that they have worked on, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

manufacturer specifications for vehicle, vessel or machinery charging systems

two different vehicles, vessels or machinery with charging system faults

diagnostic equipment for vehicle, vessel or machinery charging systems, including:

multimeter

scan tool

tools, equipment and materials appropriate for repairing vehicle, vessel or machinery charging systems.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1