# Assessment Requirements for AURETR031 Diagnose and repair ignition systems

Release: 1

Assessment Requirements for AURETR031 Diagnose and repair ignition systems

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standard defined in the unit’s elements and performance criteria, range of conditions and foundation skills:

diagnose and repair a fault in the ignition systems of two different vehicles, vessels or machinery, including faults in two of the following:

magneto with contact points

magneto with solid state trigger

capacitive discharge ignition

distributor with contact points

distributor with hall effect sensor

distributor with induction magnetic trigger

distributor with optical sensor

waste spark

coil on plug.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to diagnosing and repairing ignition systems, including procedures for:

using specialised tools and equipment

using appropriate personal protective equipment (PPE)

identifying hazards and controlling risks associated with:

working on high voltage ignition systems

wearing jewellery while working around high current wiring systems

operating principles of ignition systems and associated components, including:

generating principles, including Faraday’s law and inducing an electromotive force (EMF)

application, purpose and operation of ignition systems and components, including:

ignition coils, including:

internal component function and operation: primary windings, iron core and secondary windings

electromagnetic induction in the coil

spark plugs: identification, thread size, reach, heat range, resistance-type plugs, and type and number of earth electrodes

ignition leads

Kettering ignition system, including:

primary section: battery, ignition switch, ballast resistor, ignition coil primary windings, contact breaker point and capacitor

secondary section: ignition coil secondary windings, coil high tension lead, rotor button and distributor cap, spark plug high tension leads and spark plugs

firing order

ignition timing and methods of varying timing according to engine load and speed

electronic ignition systems, including:

pulse generator systems: stator and rotor units, ignition control units, current limiting and dwell control

Hall effect systems: Hall effect device, and permanent magnet and signal waveform

optical ignition systems: light emitting diodes (LEDs), rotating disc and optical electronic semiconductors

magneto ignition systems: shuttle and inductor magnetos, and energy transfer ignition systems

capacitive discharge ignition (CDI) systems:

transformer, charging circuit, triggering circuit, main capacitor and rectifier

alternating current CDI

direct current CDI

waste spark ignition systems

coil on plug ignition systems

diagnostic testing procedures for ignition systems, including:

using diagnostic flow charts

testing electrical systems, including procedures for:

accessing electrical terminals and using test probes without damaging connectors, fuse holders or wiring

checking resistance, current flow and voltage drop of ignition system circuits

testing ignition coils

using oscilloscopes, including interpreting ignition system primary and secondary waveforms

repair procedures for ignition systems, including:

removing and replacing faulty or damaged components

setting contact breaker gap and checking dwell

adjusting ignition system timing

removing, replacing and re-timing distributors

post-repair testing procedures for ignition systems, including static and dynamic performance tests of ignition systems.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the ignition systems that they have worked on, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

manufacturer ignition system specifications

two different vehicles, vessels or machinery with ignition system faults

diagnostic equipment for ignition systems, including:

multimeter

scan tool

oscilloscope

tools, equipment and materials appropriate for repairing vehicle, vessel or machinery ignition systems.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1