# Assessment Requirements for AURETR112 Test and repair basic electrical circuits

Release: 1

Assessment Requirements for AURETR112 Test and repair basic electrical circuits

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with AUR Automotive Retail, Service and Repair Training Package Version 6.0 |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

test the following basic electrical circuits:

one short circuit across an electrical resistive load

one open circuit before or after an electrical resistive load

one high resistance circuit before or after an electrical resistive load

current flow across an electrical resistive load

voltage drop across an electrical resistive load

test for continuity in a basic wiring harness and visually check terminals in the wiring harness for moisture ingress and corrosion

carry out four different circuit repairs in which the work must involve:

crimping various terminal sizes

soldering wire gauges to terminals

applying heat shrink insulation to crimped or soldered joints

replacing male and female terminals in a connector housing

performing terminal retention checks to replaced terminals.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

methods to locate and interpret information required to test and repair basic electrical circuits, including:

technical information, including:

circuit types

circuit diagrams

circuit symbols

manufacturer specifications and procedures or equivalent documentation

workplace procedures required to test and repair basic electrical circuits, including:

establishing serviceability of tools and equipment

documentation procedures

housekeeping procedures, including:

examination of tools and equipment

storage of equipment

identification, tagging and isolation of faulty equipment

disposal of excess materials

recycling procedures

workplace health and safety (WHS) requirements relating to testing and repairing basic electrical circuits, including procedures for:

using specialised tools and equipment

selecting and using personal protective equipment (PPE)

identifying hazards and controlling risks associated with wearing jewellery while working around electrical systems

environmental procedures relating to testing and repairing basic electrical circuits

basic theory and principles of automotive electrical systems, including:

current

alternating current (AC)

direct current (DC)

voltage

resistance

power

Ohm’s law

identification and operation of the following electrical circuits:

series circuits

parallel circuits

series parallel circuits

key features of basic electrical circuit components, including:

cable types, including:

cable sizes

current carrying capacity

circuit protection devices

switches

relays

automotive globes

procedures for using and operating electrical test equipment, including:

digital multimeters

test lights suitable for circuit testing, including resistive and light emitting diode (LED)

test probes suitable for testing circuit continuity

fused test lead

common types of faults, including:

open circuits

high resistance circuits

short circuits

damaged insulation

frayed wires

burnt wiring

water and moisture ingress

connector damage

terminal damage

testing procedures for electrical systems, including:

accessing electrical terminals and using test probes without damaging connectors, fuse holders or wiring

electrical measuring and testing procedures, including:

electrical resistance checks

open and short circuit tests

voltage drop tests

current flow tests

visual testing procedures, including:

component moisture ingress

connector damage

repair procedures for electrical circuits, including:

wire soldering procedures

terminal crimping procedures

replacing male and female terminals in a connector housing

connector removal and replacement procedures

basic electrical components removal and replacement procedures

post-repair testing procedures, including:

confirming that electrical system is operating to manufacturer specifications

confirming that no other problems are present as a result of the repair.

# Assessment Conditions

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the basic electrical circuits they have tested and repaired, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

manufacturer electrical system specifications

vehicle or machinery with faults in the basic electrical circuits specified in the performance evidence

electrical test equipment to test basic electrical circuits and components

consumable materials relevant to repairing basic electrical circuits and components, including connectors, terminals, wire, electrical tape and heat shrink sleeving

electrical repair tools, equipment and materials relating to basic electrical circuits and components.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1