# Assessment Requirements for AURETR120 Diagnose and repair network electronic control systems

Release: 1

Assessment Requirements for AURETR120 Diagnose and repair network electronic control systems

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with AUR Automotive Retail, Service and Repair Training Package Version 6.0 |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

diagnose and repair a fault in at least three different network electronic control circuits that set network communication diagnostic trouble codes (DTCs), including one single wire circuit (LIN-bus) and one two-wire circuit (CAN-bus).

carry out a diagnostic test in the course of the above work for at least one of the following faults:

high resistance in an input system

worn or damaged wiring or connectors

shorted system components.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

methods to locate and interpret information required to diagnose and repair network electronic control systems, including:

information provided by customers and supervisors

manufacturer specifications and procedures or equivalent documentation

workplace procedures required to diagnose and repair network electronic control systems, including:

establishing serviceability of tools and equipment

documentation procedures

housekeeping procedures, including:

examination of tools and equipment

storage of equipment

identification, tagging and isolation of faulty equipment

disposal of excess materials

recycling procedures

workplace health and safety (WHS) requirements relating to diagnosing and repairing network electronic control systems, including procedures for identifying hazards and controlling risks associated with:

working on vehicle high voltage ignition systems

wearing jewellery while working around high current wiring systems

environmental procedures relating to diagnosing and repairing network electronic control systems.

diagnostic testing procedures for network electronic control systems, including:

using diagnostic flow charts

accessing and interpreting scan tool system data, including:

DTCs, including ‘U’ type communication codes

live data

freeze frame data

waveforms

testing electrical systems, including procedures for:

accessing electrical terminals and using test probes without damaging connectors, fuse holders or wiring

determining damage to system wiring and connectors

repair procedures for network electronic control systems, including:

connector removal and replacement procedures

removal and replacement procedures for vehicle network electronic control system components

post-repair testing procedures for network electronic control systems, including:

confirming fault rectification

DTC clearing procedures

checking for electrical connector mating

operating principles of network electronic control systems and associated components, including:

assurance of message delivery, non-conflicting messages, minimum time of delivery, and electromagnetic field (EMF) noise resilience

network topographies

network protocols, including characteristics and data speeds

purpose and operation of network electronic control systems and components, including:

controlled area network (CAN), including:

nodes: host processor, CAN controller, and transceiver

gateway modules

terminating resistors

data transmission

vehicle data logic connector (DLC)

local interconnect network (LIN), including:

master and slaves

data transmission.

# Assessment Conditions

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the network electronic control systems that they have worked on, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

manufacturer specifications for network electronic control systems

two different network electronic control circuits

diagnostic equipment for network electronic control circuits, including:

multimeter

scan tool

oscilloscope

tools, equipment and materials appropriate for repairing network electronic control circuits.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1