# Assessment Requirements for AURETR236 Diagnose and repair electronically controlled suspension systems

Release: 1

Assessment Requirements for AURETR236 Diagnose and repair electronically controlled suspension systems

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with AUR Automotive Retail, Service and Repair Training Package Version 6.0 |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

diagnose and repair a fault in the following electronically controlled suspension system components of at least two different vehicles or machinery:

ride height sensor circuits

wheel speed sensor circuits

steering rate sensor circuits

ride height control relay circuits

air compressor motor control circuits

air spring fill-vent solenoid control valve circuits

remove, refit or replace either a solenoid actuated shock absorber or a load sensing shock absorber from an electronically controlled suspension system in one of the above vehicles or machinery, and retest the system

carry out a diagnostic test in the course of the above for at least one of the following faults:

high resistance in an input system

loose or damaged connectors or wiring

damaged control sensors.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

methods to locate and interpret information required to diagnose and repair electronically controlled suspension systems including:

information provided by customers and supervisors

manufacturer specifications and procedures or equivalent documentation

workplace procedures required to diagnose and repair electronically controlled suspension systems, including:

establishing serviceability of tools and equipment

documentation procedures

housekeeping procedures, including:

examination of tools and equipment

storage of equipment

identification, tagging and isolation of faulty equipment

disposal of excess materials

recycling procedures

workplace health and safety (WHS) requirements relating to diagnosing and repairing electronically controlled suspension systems, including procedures for:

using specialised tools and equipment

using appropriate personal protective equipment (PPE)

identifying hazards and controlling risks associated with:

working with stored energy in springs and torsion bars, including procedures for removing tension from suspension components

manual handling heavy suspension system components

principles of electronically controlled suspension systems and associated components, including:

vehicle ride and handling

typical vehicle or machinery equipped with electronically controlled suspension systems

purpose and operation of electronically controlled suspension systems and components, including:

adaptive suspension control module

control sensors and actuators

suspension control module functions, including:

control of air spring settings

shock damper settings

air compressor operation

diagnostic testing procedures for electronically controlled suspension systems, including:

accessing and interpreting scan tool system data, including:

diagnostic trouble codes (DTCs)

live data

waveforms

using diagnostic flow charts

testing electrical systems, including procedures for:

accessing electrical terminals and using test probes without damaging connectors, fuse holders or wiring

resistance and voltage drop tests

open and short circuit tests

checking shorts to signal, power circuits and grounds

repair procedures for electronically controlled suspension systems, including procedures for removing and replacing system components

post-repair testing procedures for electronically controlled suspension systems, including:

DTC clearing procedures

static and dynamic performance tests of suspension system.

# Assessment Conditions

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the electronically controlled suspension systems that they have worked on, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

manufacturer specifications for electronically controlled suspension systems

two different vehicles or machinery with faults in their electronically controlled suspension systems

diagnostic equipment for electronically controlled suspension systems, including:

multimeter

scan tool

tools, equipment and materials appropriate for repairing electronically controlled suspension systems.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1