# Assessment Requirements for AURETU105 Retrofit automotive air conditioning and HVAC systems

Release: 1

Assessment Requirements for AURETU105 Retrofit automotive air conditioning and HVAC systems

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with AUR Automotive Retail, Service and Repair Training Package Version 6.0 |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

retrofit the air conditioning and HVAC systems of two different vehicles or machinery, in which the work must include changing the type of refrigerant.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

methods to locate and interpret information required to retrofit automotive air conditioning and HVAC systems, including:

air conditioning and HVAC system manufacturer specifications

Australian automotive code of practice: control of refrigerant gases during manufacture, installation, servicing or de-commissioning of motor vehicle air conditioners

workplace procedures required to retrofit automotive air conditioning and HVAC systems, including:

establishing the serviceability of tools and equipment

documentation procedures

housekeeping procedures, including:

examination of tools and equipment

storage of equipment

identification, tagging and isolation of faulty equipment

safe disposal of materials

recycling procedures

workplace health and safety (WHS) requirements relating to retrofitting air conditioning and HVAC systems, including procedures for:

working with refrigerants at boiling point given risk of frostbite

working with system lubricants, including carcinogenic oils

identifying refrigerant and oil type, including:

service decal sticker

testing refrigerant

types and location of service ports

handling, storing and transporting refrigerant cylinders and recovery cylinders, including:

pressure ratings

pressure relief devices

outlet connection type

identifying and handling refrigerants, including hydrocarbon (HC) refrigerants

selecting and using personal protective equipment (PPE)

identifying firefighting equipment

environmental requirements, including procedures for:

preventing loss of refrigerant to the atmosphere

handling materials and refrigerant recovery equipment

key requirements of federal Ozone Protection and Synthetic Greenhouse Gas Management Regulations, including refrigerant handling licence and refrigerant trading authorisation

purpose and operation of automotive air conditioning and HVAC systems and components, including:

high pressure and low pressure sides of air conditioning systems

compressors, including:

piston, scroll and rotary vane compressors

electric compressors

variable displacement compressors

clutchless compressors electromagnetic clutches

condensers

receiver-dryers, including filters and desiccants

expansion valves, including capillary tubes

evaporators

thermostats

refrigerants, including R12, R134a, R1234yf, and hydrocarbon blends

compressor oils

air conditioner and heating controls, including levers and ducting

air conditioner and heating electrical circuits and sensors, including:

high and low pressure switches

pressure relief valves

temperature sensors

climate control systems

procedures for retrofitting and associated modifications for air conditioning and HVAC systems, including:

determining original system fitted to vehicle, including refrigerant and oil type and types of components

determining appropriate retrofit and associated modification options, including:

determining required system performance output

component replacement, including O rings and seals

refrigerant and oil selection

procedures for removing and replacing system components

procedures for changing system refrigerant, including:

changing refrigerant gas from R12 to R134a:

receiver dryer filter replacement

change system fittings for manifold gauge attachment

change oil type

reasons why R134a refrigerant must not be retrofitted to R1234yf

reasons why R134a refrigerant must not be retrofitted to blends of hydrocarbons

procedures for recovering automotive refrigerant, including:

testing refrigerant to determine its type

connecting manifold and gauge set and recovery unit, including types and location of service ports

identifying recovery cylinder appropriate to the refrigerant

operating recovery unit, including weighing recovery cylinder before after recovery

disconnecting and storing recovery unit and cylinder

procedures for re-gassing air conditioning and HVAC systems, including:

testing system for leaks using vacuum testing and dry nitrogen pressure testing

evacuating the system using a vacuum pump and vacuum gauge

using manifold gauges, refrigerant scales or charging station to charge system

post-retrofit testing procedures for air conditioning and HVAC systems, including procedures for checking for:

refrigerant leaks

heater core and system water coolant leaks

system performance is operating to manufacturer specifications

reported pre-modification issue has been resolved

no other faults are present

static and dynamic performance tests of air conditioning and HVAC systems, including checking:

ambient temperature

vent temperature

condenser and suction line temperature.

# Assessment Conditions

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the air conditioning and HVAC systems that they have retrofitted, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

vehicle or machinery air conditioning and HVAC system manufacturer specifications

Australian automotive code of practice: Control of refrigerant gases during manufacture, installation, servicing or de-commissioning of motor vehicle air conditioners

two different vehicles or machinery with air conditioning and HVAC systems requiring retrofitting

retrofit and testing equipment for air conditioning and HVAC systems, including:

manifold and gauge set

recovery unit and recovery cylinder

vacuum pump and gauge

leak detector

refrigerant analyser

refrigerant

refrigerant scales or charging station

dry nitrogen cylinder and gauge.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards; and hold an Australian Refrigerant Council (ARC) Refrigerant Handling licence.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1