# AURETU105 Retrofit automotive air conditioning and HVAC systems

Release: 1

AURETU105 Retrofit automotive air conditioning and HVAC systems

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with AUR Automotive Retail, Service and Repair Training Package Version 6.0 |

# Application

This unit describes the skills and knowledge required to retrofit automotive air conditioning systems, including heating, ventilation and air conditioning (HVAC) systems, fitted to a range of vehicles and machinery. It involves preparing for the task, selecting the correct de-gas procedure, carrying out the retrofit and associated modifications, performing post-retrofit testing, and completing workplace processes and documentation. Air conditioning systems may contain a mixture of both natural and flammable synthetic refrigerants that are difficult to accurately identify and safely recover. To minimise the possibility of ignition, the procedures and equipment used when recovering refrigerants must be those required for flammable refrigerants.

The unit applies to those who work in the automotive service and repair industry. Automotive air conditioners, including HVAC systems, include those in agricultural machinery, heavy commercial vehicles, light vehicles or mobile plant machinery.

Licensing requirements apply to this unit. Users are advised to check with relevant regulatory authority. An Australian Refrigeration Council accredited (ARCtick) Refrigerant Handling licence is required for those carrying out this work.

# Unit Sector

Electrical Technical – Air Conditioning and HVAC

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare to retrofit system | 1.1 Identify job requirements from workplace instructions 1.2 Identify required information for retrofit activity, including manufacturer specifications and Australian automotive code of practice 1.3 Identify refrigerant required according to workplace procedures and workplace health and safety and environmental requirements 1.4 Identify retrofit option required for the job 1.5 Identify hazards and environmental issues, assess potential risks and implement control measures in line with workplace procedures 1.6 Identify tools and equipment required for inspect and servicing activity and establish serviceability according to workplace procedures |
| 2. De-gas system | 2.1 De-gas system according to manufacturer specifications, Australian automotive code of practice, workplace procedures, and workplace health and safety and environmental requirements 2.2 Refrigerant is recovered into required refrigerant recovery cylinder according to manufacturer specifications, Australian automotive code of practice, workplace procedures, and workplace health and safety and environmental requirements 2.3 Measure oil recovered and determine replacement volume |
| 3. Retrofit system | 3.1 Carry out air conditioning retrofit procedures according to manufacturer specifications, Australian automotive code of practice, workplace procedures, and workplace health safety and environmental requirements 3.2 Pressure test system and confirm there are no leaks according to manufacturer specifications, Australian automotive code of practice, workplace procedures, and workplace health and safety and environmental requirements 3.3 Evacuate system according to manufacturer specifications, Australian automotive code of practice, workplace procedures, and workplace health and safety and environmental requirements 3.4 Re-gas system with refrigerant gas and lubricating oil according to manufacturer specifications, Australian automotive code of practice, and workplace procedures 3.5 Carry out post-retrofit testing according to workplace procedures |
| 4. Complete work processes | 4.1 Conduct final inspection according to workplace procedures and confirm vehicle is ready for use 4.2 Clear work area and dispose of or recycle materials according to workplace procedures 4.3 Complete Australian Refrigeration Council accredited (ARCtick) service decal sticker and workplace documentation according to workplace procedures. |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance but not explicit in the performance criteria.

| SKILL | DESCRIPTION |
| --- | --- |
| Learning | Locates required sources of information efficiently |
| Reading | Interprets text, symbols and diagrams relating to retrofit and associated modification information from manufacturer specifications, and workplace instructions and procedures Interprets key requirements of Australian automotive code of practice relating to retrofitting air conditioning and HVAC systems |
| Writing | Fills out workplace documentation legibly, including ARCtick stickers, when reporting test findings, making retrofit recommendations, and recording parts and material used |
| Oral communication | Listens to workplace instructions and asks questions to clarify job requirements |
| Numeracy | Matches refrigerant types and identification numbers to workplace instructions, vehicle and component part lists, and code of practice requirements Interprets weighting measurements, including tare and gross weights Interprets readings on digital and analogue pressure gauges Measures temperatures and pressures, and uses basic mathematical operations, including addition and subtraction, to calculate deviations from manufacturer specifications |
| Planning and organising | Plans own work requirements Prioritises actions to achieve required outcomes Ensures tasks are completed within workplace timeframes |
| Technology | Uses workplace technology and tools. |

# Unit Mapping Information

Supersedes and is equivalent to AURETU005 Retrofit and modify air conditioning and HVAC systems.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Assessment Requirements for AURETU105 Retrofit automotive air conditioning and HVAC systems

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with AUR Automotive Retail, Service and Repair Training Package Version 6.0 |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

retrofit the air conditioning and HVAC systems of two different vehicles or machinery, in which the work must include changing the type of refrigerant.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

methods to locate and interpret information required to retrofit automotive air conditioning and HVAC systems, including:

air conditioning and HVAC system manufacturer specifications

Australian automotive code of practice: control of refrigerant gases during manufacture, installation, servicing or de-commissioning of motor vehicle air conditioners

workplace procedures required to retrofit automotive air conditioning and HVAC systems, including:

establishing the serviceability of tools and equipment

documentation procedures

housekeeping procedures, including:

examination of tools and equipment

storage of equipment

identification, tagging and isolation of faulty equipment

safe disposal of materials

recycling procedures

workplace health and safety (WHS) requirements relating to retrofitting air conditioning and HVAC systems, including procedures for:

working with refrigerants at boiling point given risk of frostbite

working with system lubricants, including carcinogenic oils

identifying refrigerant and oil type, including:

service decal sticker

testing refrigerant

types and location of service ports

handling, storing and transporting refrigerant cylinders and recovery cylinders, including:

pressure ratings

pressure relief devices

outlet connection type

identifying and handling refrigerants, including hydrocarbon (HC) refrigerants

selecting and using personal protective equipment (PPE)

identifying firefighting equipment

environmental requirements, including procedures for:

preventing loss of refrigerant to the atmosphere

handling materials and refrigerant recovery equipment

key requirements of federal Ozone Protection and Synthetic Greenhouse Gas Management Regulations, including refrigerant handling licence and refrigerant trading authorisation

purpose and operation of automotive air conditioning and HVAC systems and components, including:

high pressure and low pressure sides of air conditioning systems

compressors, including:

piston, scroll and rotary vane compressors

electric compressors

variable displacement compressors

clutchless compressors electromagnetic clutches

condensers

receiver-dryers, including filters and desiccants

expansion valves, including capillary tubes

evaporators

thermostats

refrigerants, including R12, R134a, R1234yf, and hydrocarbon blends

compressor oils

air conditioner and heating controls, including levers and ducting

air conditioner and heating electrical circuits and sensors, including:

high and low pressure switches

pressure relief valves

temperature sensors

climate control systems

procedures for retrofitting and associated modifications for air conditioning and HVAC systems, including:

determining original system fitted to vehicle, including refrigerant and oil type and types of components

determining appropriate retrofit and associated modification options, including:

determining required system performance output

component replacement, including O rings and seals

refrigerant and oil selection

procedures for removing and replacing system components

procedures for changing system refrigerant, including:

changing refrigerant gas from R12 to R134a:

receiver dryer filter replacement

change system fittings for manifold gauge attachment

change oil type

reasons why R134a refrigerant must not be retrofitted to R1234yf

reasons why R134a refrigerant must not be retrofitted to blends of hydrocarbons

procedures for recovering automotive refrigerant, including:

testing refrigerant to determine its type

connecting manifold and gauge set and recovery unit, including types and location of service ports

identifying recovery cylinder appropriate to the refrigerant

operating recovery unit, including weighing recovery cylinder before after recovery

disconnecting and storing recovery unit and cylinder

procedures for re-gassing air conditioning and HVAC systems, including:

testing system for leaks using vacuum testing and dry nitrogen pressure testing

evacuating the system using a vacuum pump and vacuum gauge

using manifold gauges, refrigerant scales or charging station to charge system

post-retrofit testing procedures for air conditioning and HVAC systems, including procedures for checking for:

refrigerant leaks

heater core and system water coolant leaks

system performance is operating to manufacturer specifications

reported pre-modification issue has been resolved

no other faults are present

static and dynamic performance tests of air conditioning and HVAC systems, including checking:

ambient temperature

vent temperature

condenser and suction line temperature.

# Assessment Conditions

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the air conditioning and HVAC systems that they have retrofitted, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

vehicle or machinery air conditioning and HVAC system manufacturer specifications

Australian automotive code of practice: Control of refrigerant gases during manufacture, installation, servicing or de-commissioning of motor vehicle air conditioners

two different vehicles or machinery with air conditioning and HVAC systems requiring retrofitting

retrofit and testing equipment for air conditioning and HVAC systems, including:

manifold and gauge set

recovery unit and recovery cylinder

vacuum pump and gauge

leak detector

refrigerant analyser

refrigerant

refrigerant scales or charging station

dry nitrogen cylinder and gauge.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards; and hold an Australian Refrigerant Council (ARC) Refrigerant Handling licence.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1