# AURHTB007 Diagnose and repair heavy commercial vehicle electronic braking systems

Release: 1

AURHTB007 Diagnose and repair heavy commercial vehicle electronic braking systems

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Application

This unit describes the performance outcomes required to diagnose and repair faults in the electronically controlled braking systems of heavy commercial vehicles. It involves preparing for the task, selecting the correct diagnostic procedure, carrying out the diagnosis and the repair, performing post-repair testing, and completing workplace processes and documentation.

It applies to those working in the automotive service and repair industry. The braking systems include those in heavy commercial vehicles. This unit does not apply to agricultural machinery or mobile plant machinery.

No licensing, legislative, regulatory or certification requirements apply to this unit at the time of publication.

# Competency Field

Mechanical - Heavy Vehicle

# Unit Sector

Technical - Brakes

# Elements and Performance Criteria

| Elements Elements describe the essential outcomes. | Performance Criteria Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold and italicised text is used, further information is detailed in the range of conditions section. |
| --- | --- |
| 1. Prepare to diagnose heavy commercial vehicle electronic braking system | 1.1 Job requirements are determined from workplace instructions 1.2 Diagnostic information is sourced and interpreted 1.3 Diagnostic options are analysed and those most appropriate to the circumstances are selected 1.4 Hazards associated with the work are identified and risks are managed 1.5 Diagnostic tools and equipment are selected and checked for serviceability |
| 2. Diagnose electronic braking system | 2.1 Diagnostic tests are carried out according to manufacturer specifications, workplace procedures and safety requirements 2.2 Faults are identified from diagnostic test results and causes of faults are determined 2.3 Diagnosis findings, including recommendations for necessary repairs or adjustments, are reported according to workplace procedures |
| 3. Repair electronic braking system | 3.1 Repair information is sourced and interpreted 3.2 Repair options are analysed and those most appropriate to the circumstances are selected 3.3 Repair tools, equipment and materials are selected and checked 3.4 Repairs and component replacements and adjustments are carried out according to manufacturer specifications, workplace procedures and safety requirements, and without causing damage to components or systems 3.5 Post-repair testing is carried out and diagnostic fault codes are confirmed as having have been rectified according to workplace procedures to confirm fault rectification, and any further problems detected as having been introduced during the repair process are rectified |
| 4. Complete work processes | 4.1 Final inspection is made to ensure work is to workplace expectations and vehicle is presented ready for use 4.2 Work area is cleaned, waste and non-recyclable materials are disposed of, and recyclable material is collected 4.3 Tools and equipment are checked and stored according to workplace procedures 4.4 Workplace documentation is processed according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance and are not explicit in the performance criteria.

| Skills | Description |
| --- | --- |
| Learning skills to: | locate appropriate sources of information efficiently. |
| Reading skills to: | interpret information from manufacturer and workshop literature when seeking braking system specifications and procedures. |
| Writing skills to: | legibly and accurately fill out workplace documentation when reporting diagnostic findings, making repair recommendations, and recording parts and material used. |
| Oral communication skills to: | clarify instructions report diagnostic findings and make repair recommendations. |
| Numeracy skills to: | interpret electrical measurements and readings on diagnostic test equipment measure braking system components and use basic mathematical operations, including addition and subtraction, to calculate distances, tolerances and deviations from manufacturer specifications. |
| Planning and organising skills to: | plan own work requirements and prioritise actions to achieve required outcomes and ensure tasks are completed within workplace timeframes. |
| Technology skills to: | use precision measuring and diagnostic tools and equipment. |

# Range of Conditions

This section specifies work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. Bold italicised wording, if used in the performance criteria, is detailed below.

| Safety requirements must include: | work health and safety (WHS) and occupational health and safety (OHS) requirements, including procedures for working safely with high air pressure systems. |
| --- | --- |

# Unit Mapping Information

Equivalent to AURHTB3007 Diagnose and repair heavy vehicle electronic braking systems

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Assessment Requirements for AURHTB007 Diagnose and repair heavy commercial vehicle electronic braking systems

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standards defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

diagnose and repair a fault in the electronic components of two different heavy commercial vehicle electronic braking systems.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to diagnosing and repairing heavy commercial vehicle electronic braking systems, including procedures for working safely with high air pressure systems

operating principles of heavy commercial vehicle electronic braking systems and associated components, including:

electronic systems

pneumatic systems

Australian Design Rules ADR35 and ADR38 relating to repairing heavy vehicle brakes

application, purpose and operation of heavy commercial vehicle electronic braking systems and components, including:

electronic control unit (ECU)

dual circuit pneumatic foot pedal control valve with electronic position sensing

front circuit electric over air modulation and relay valve

rear circuit electric over air modulation and relay valve

wheel speed sensors and pole wheel

tractor protection air valve

pressure sensor in airbag system or load sensor

diagnostic testing procedures for heavy commercial vehicle electronic braking systems that comply with manufacturer specifications, including:

accessing and interpreting scan tool system data, including:

diagnostic trouble codes (DTCs)

live data

freeze frame data

waveforms

repair procedures for heavy commercial vehicle electronic braking systems that comply with manufacturer specifications, including removal, replacement and adjustment procedures for electronic braking system components

post-repair testing procedures for heavy commercial vehicle electronic braking systems that comply with manufacturer specifications, including static and dynamic performance tests of electronic braking systems.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the heavy vehicle electronic braking systems that they have worked on, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

manufacturer braking system specifications

two different heavy commercial vehicles with faults in their electronic braking systems

diagnostic equipment for heavy commercial vehicle electronic braking systems

Australian Design Rules ADR35 and ADR38

tools, equipment and materials appropriate for repairing and adjusting heavy commercial vehicle electronic braking systems.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1