# Assessment Requirements for AURHTE001 Remove and install heavy vehicle engine assemblies

Release: 1

Assessment Requirements for AURHTE001 Remove and install heavy vehicle engine assemblies

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standards defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

remove and install two heavy vehicle engines, in which the work must involve:

removing and refitting ancillary components and systems

adjusting oil and fluid levels

post-installation adjustments

post-installation testing.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to removal and installation of heavy vehicle engines, including:

isolating and stabilising vehicle or machinery

using vehicle or machinery lifting devices and supporting equipment’

working with compression ignition engines

environmental requirements, including procedures for trapping, storing and disposing of oils, fluids, coolants and gases released during removal and installation of engine assemblies

methods of disconnecting and reconnecting related systems, including:

air conditioning system

cooling system

electrical systems

exhaust system

fuel system

power steering system

hydraulic system

transmission systems, including:

automatic transmission

manual transmission

removal procedures for heavy vehicle engine assemblies, including:

correct use of engine lifting devices and supporting equipment

item tagging procedures

methods for correctly disposing of hazardous substances

installation procedures for heavy vehicle engine assemblies

post-installation testing and adjustment procedures for heavy vehicle engine assemblies, including:

engine pre-start procedures

operational testing and run-in procedures

engine adjustments and cylinder head bolt retensioning as required

throttle adjustments

coolant adjustments

ancillary engine systems.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the heavy vehicle engine assemblies that they have removed and installed, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

manufacturer engine specifications

two different heavy vehicles requiring removal and installation of engine assemblies

tools, equipment and materials appropriate for removing and replacing heavy vehicle engines, including engine lifting devices and supporting equipment.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1