# Assessment Requirements for AURJTB101 Diagnose and repair motorcycle braking systems

Release: 1

Assessment Requirements for AURJTB101 Diagnose and repair motorcycle braking systems

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with AUR Automotive Retail, Service and Repair Training Package Version 6.0 |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

diagnose and repair a fault in the following two braking systems of at least two different motorcycles, each time repairing a different system from the following:

at least one hydraulically-operated two-wheel disc braking system, in which the work must involve:

removing, bleeding and adjusting hand brake lever and brake pedal, master cylinder, brake hose, brake callipers, brake pads and brake rotors

refitting or replacing them

at least one mechanically-operated drum braking system, in which the work must involve:

removing and adjusting hand brake lever and brake pedal, cables and rods, brake drums and shoes, and actuating mechanisms

refitting or replacing them.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

methods to locate and interpret information required to diagnose and repair motorcycle braking systems, including:

information provided by customers and supervisors

manufacturer specifications and procedures or equivalent documentation

workplace procedures required to diagnose and repair motorcycle braking systems, including:

establishing serviceability of tools and equipment

documentation procedures

housekeeping procedures, including:

examination of tools and equipment

storage of equipment

identification, tagging and isolation of faulty equipment

disposal of excess materials

recycling procedures

workplace health and safety (WHS) requirements relating to diagnosing and repairing motorcycle braking systems, including procedures for:

lifting and supporting motorcycles

handling and controlling brake dust and brake fluids

environmental requirements relating to diagnosing and repairing motorcycle braking systems, including procedures for trapping, storing and disposing of brake dust and brake fluid released from motorcycle braking systems

operating principles of motorcycle braking systems, including:

levers

friction

hydraulics, including the relationship between force, pressure and area

properties of brake fluids, including:

Department of Transportation (DOT) classification

synthetic and non-synthetic

compatibility of fluid types

purpose and operation of motorcycle braking systems and components, including:

master cylinders

brake fluids

disc brake systems, including:

types of brake discs

disc pads

self-adjustment of disc pads

fixed, floating and multi-piston brake callipers

drum brake systems, including:

self-energising, servo effect

mechanically operated systems

hydraulically operated systems

manual and self-adjustment systems

combined braking systems

diagnostic testing procedures for motorcycle braking systems, including procedures for:

operational tests of master cylinder

testing brake fluid

operational tests of hand brake

operational tests of drum and disc brakes

measuring and evaluating brake drums, shoes, discs and pads

repair procedures for motorcycle braking systems, including procedures for:

removing, replacing and adjusting braking system components, including master cylinder, switches, brake hoses and lines, rods and cables, disc callipers and disc pads, brake drums, brake shoes and wheel cylinders

removing, replacing and adjusting braking system components, including brake lever, rods and cables, brake drums and brake shoes

bleeding brakes

post-repair testing procedures for motorcycle braking systems, including procedures for road testing and bedding in brakes.

# Assessment Conditions

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the motorcycle braking systems that they have worked on, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

motorcycle repair workplace or simulated workplace

workplace instructions

manufacturer motorcycle braking system specifications

two different motorcycles with faults in the braking systems specified in the performance evidence

diagnostic equipment for motorcycle braking systems

tools, equipment and materials appropriate for repairing motorcycle braking systems.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1