# Assessment Requirements for AURJTX002 Diagnose and repair motorcycle manual transmissions

Release: 1

Assessment Requirements for AURJTX002 Diagnose and repair motorcycle manual transmissions

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standards defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

diagnose and repair a fault in two different motorcycles with different manual transmissions, in which the work must involve removing, dismantling, reassembling and refitting the transmissions.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to diagnosing and repairing motorcycle manual transmissions, including procedures for handling transmission fluids and clutch materials

environmental requirements, including procedures for trapping, storing and disposing of fluids released from transmissions

operating principles of motorcycle manual transmissions and associated components, including:

gears types

gear ratios

application, purpose and operation of motorcycle manual transmissions and components, including:

transmission types, including:

vertical split-type crankcase

horizontal split-type crankcase

cassette-type

sliding gear transmissions, including:

dog clutches

shifter drums

cam plates

constant-mesh transmissions, including ball lock-type systems

quick shifters

types and compatibility of transmission system oils and additives

diagnostic testing procedures for motorcycle manual transmissions, including procedures for analysing:

component wear

abnormal system noise

repair procedures for motorcycle manual transmissions, including procedures for removing, dismantling, reassembling, refitting and adjusting components

post-repair testing procedures for motorcycle manual transmissions.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the motorcycle manual transmissions that they have worked on, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

motorcycle repair workplace or simulated workplace

workplace instructions

manufacturer motorcycle manual transmission specifications

two different motorcycles with different manual transmissions with faults

diagnostic equipment for motorcycle manual transmissions

tools, equipment and materials appropriate for repairing motorcycle manual transmissions.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1