# Assessment Requirements for AURKTA009 Diagnose complex faults in mobile plant hydraulic systems

Release: 1

Assessment Requirements for AURKTA009 Diagnose complex faults in mobile plant hydraulic systems

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standards defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

diagnose a complex fault in the hydraulic systems of three different mobile plant machinery

the above diagnosis must involve two of the following types of complex faults:

an intermittent fault

a fault that affects more than one system

a fault introduced as a result of a system repair

an indirect fault caused by the influence of external systems.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to diagnosing complex faults in mobile plant hydraulic systems, including procedures for:

managing stored energy in springs and accumulators

working with high pressure fluid hazards

tagging out and isolating machines, and wheel chocking

environmental requirements, including procedures for trapping, storing and disposing of fluid released from hydraulic systems

types of complex faults relating to mobile plant hydraulic systems, including:

intermittent

multi-system

introduced as a result of system repair

indirect, caused by the influence of external systems

requirements of AS 1101.1 Graphic symbols for general engineering, Part 1: Hydraulic and pneumatic systems

types, function and operation of mobile plant hydraulic systems, including:

reservoirs

accumulators

pumps

valves

actuators

motors

filters

conductors and connectors

testing procedures for mobile plant hydraulic systems, including:

abnormal noise analysis

implement creep

oil flow and pressure testing

excessive internal leakage in both actuators and pumps

component failure analysis

types, functions, operation and limitations of diagnostic testing equipment required to diagnose complex faults in mobile plant hydraulic systems, including:

scan tool

oil pressure gauge

temperature measuring equipment

flow meter

procedures for accessing and interpreting scan tool system data, including:

diagnostic trouble codes (DTCs), including:

conditions that set the DTCs

conditions for running DTCs

live data

freeze frame data

waveforms

continuous and non-continuous monitored systems

methods and processes for documenting and reporting diagnostic findings and recommendations.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the mobile plant hydraulic systems that they have worked on, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

manufacturer mobile plant hydraulic system specifications

hydraulic systems of three different mobile plant machinery with complex faults

mobile plant hydraulic system diagnostic equipment, including:

scan tool

flow meter

oil pressure gauge

temperature measuring equipment

AS 1101.1 Graphic symbols for general engineering, Part 1: Hydraulic and pneumatic systems

tools, equipment and materials appropriate for diagnosing complex faults in mobile plant hydraulic systems.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1