# Assessment Requirements for AURLTB103 Diagnose and repair light vehicle hydraulic braking systems

Release: 1

Assessment Requirements for AURLTB103 Diagnose and repair light vehicle hydraulic braking systems

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with AUR Automotive Retail, Service and Repair Training Package Version 6.0 |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

diagnose and repair a fault in:

two different light vehicle four-wheel disc hydraulic braking systems, in which the work must involve removing and refitting or replacing the brake callipers, disc pads and disc rotors

one light vehicle drum braking system, in which the work must involve removing and refitting or replacing brake drums, brake shoes and wheel cylinders

one light vehicle brake master cylinder, in which the work must involve removing and refitting or replacing the brake master cylinder

carry out diagnostic tests in each of the above tasks for at least one of the following faults:

worn brake pads

malfunctioning master cylinders

contaminated brake fluid.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

methods to locate and interpret information required to diagnose and repair light vehicle hydraulic braking systems, including:

information provided by customers and supervisors

manufacturer specifications and procedures or equivalent documentation

workplace procedures required to diagnose and repair light vehicle hydraulic braking systems, including:

establishing serviceability of tools and equipment

documentation procedures

housekeeping procedures, including:

examination of tools and equipment

storage of equipment

identification, tagging and isolation of faulty equipment

disposal of excess materials

recycling procedures

workplace health and safety (WHS) requirements relating to diagnosing and repairing light vehicle hydraulic braking systems, including procedures for:

lifting and supporting light vehicles

handling and controlling brake dust and brake fluids

environmental requirements relating to diagnosing and repairing light vehicle hydraulic braking systems, including procedures for trapping, storing and disposing of brake dust and brake fluid released from hydraulic braking systems

operating principles of light vehicle hydraulic braking systems, including:

levers

friction

hydraulics, including the relationship between force, pressure and area

properties of brake fluids, including:

Department of Transportation (DOT) classification

synthetic and non-synthetic

compatibility of fluid types

purpose and operation of light vehicle hydraulic braking systems and components, including:

front and rear split systems

diagonal split systems

master cylinders, including:

tandem master cylinders

compensating-type master cylinders

centre-valve master cylinders

drum braking system and components, including:

self-energising or servo effect

leading and trailing shoe braking systems

duo-servo braking systems

two-leading shoe braking systems

wheel cylinders

manual and self-adjustment systems of drum braking systems

disc braking systems and components, including:

types of brake discs

disc pads

self-adjustment of disc pads

fixed, floating and dual-piston brake callipers

parking brake systems and components, including:

parking brake levers

drum parking brakes

disc parking brakes

electronic parking brake systems

braking system valves and switches, including:

pressure differential valves and switches

proportioning valves

load-sensing proportioning valves

drum brake check valves

fluid-level warning devices

vacuum operated brake boosting systems

diagnostic testing procedures for light vehicle hydraulic braking systems, including procedures for:

checking brake pedal

operational tests of brake booster

operational tests of master cylinder

testing brake fluid

operational tests of hand brake

operational tests of drum and disc brakes

measuring and evaluating brake drums, shoes, discs and pads

repair procedures for light vehicle hydraulic braking systems, including procedures for:

removing, replacing and adjusting hydraulic system components, including master cylinder, proportioning valves, switches, brake hoses and lines, disc callipers and wheel cylinders

removing, replacing and adjusting braking system components, including brake booster, master cylinder, hand brake lever and cables, brake shoes and drums, brake pads and brake discs

bleeding brakes, including anti-lock braking systems (ABS)

post-repair testing procedures for light vehicle hydraulic braking systems, including procedures for road testing and bedding in brakes.

# Assessment Conditions

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the light vehicle hydraulic braking systems that they have worked on, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

manufacturer braking system specifications

two different light vehicles with four-wheel hydraulic disc brake systems with faults

one light vehicle with a rear drum brake system

diagnostic equipment for light vehicle hydraulic braking systems

tools, equipment and materials appropriate for repairing light vehicle braking systems.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1