# Assessment Requirements for AURLTD003 Reset steering system alignment adjustments to customer specifications

Release: 1

Assessment Requirements for AURLTD003 Reset steering system alignment adjustments to customer specifications

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standards defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

reset the steering system alignment of two different vehicles, in which the work must involve measuring, recording and adjusting the following angles on all four wheels of each vehicle:

camber

caster

steering axis inclination (SAI)

toe (toe-in and toe-out)

toe-out on turns.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to resetting steering system alignment, including procedures for using air jacks

application, purpose and function of steering angles, including:

Ackermann principle of steering

camber

caster

SAI

thrust angle

toe-in and toe-out

toe-out on turns

included angle

point of intersection

scrub radius

reasons for making non-standard alignment settings on driving and ride characteristics, including:

fitting modified road springs

fitting revised shock absorbers

fitting modified sway bars

altering ride height

fitting non-standard wheel and tyre combinations

variations to understeer and oversteer

off-road use

vehicle wheel alignment pre-check procedures, including:

assessing steering system alignment adjustments against safety and Australian Design Rule (ADR) compliance

road testing

analysing tyre wear

determining wheel run-out

determining vehicle height

checking frame alignment

types and operation of wheel alignment systems, including:

laser aligners

visual aligners

camera aligners

wheel alignment checking procedures, including:

checking caster

checking camber

checking toe-in

checking toe-out on turns

wheel alignment adjustment procedures, including:

adjusting caster

adjusting camber

adjusting toe-in

adjusting toe-out on turns.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the steering systems that they have worked on, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

manufacturer steering system alignment specifications

Australian Design Rules

two different vehicles requiring steering system adjustments to the angles specified in the performance evidence

wheel alignment equipment

tools, equipment and materials appropriate for adjusting steering system alignment.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1