# Assessment Requirements for AURLTD004 Diagnose and repair light vehicle steering systems

Release: 1

Assessment Requirements for AURLTD004 Diagnose and repair light vehicle steering systems

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standards defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

diagnose and repair a fault in a light vehicle power-assisted rack and pinion steering assembly

diagnose and repair a fault in a light vehicle power steering pump

diagnose and repair a fault in one of the following light vehicle steering components:

power steering hose

tie-rod end.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to diagnosing and repairing light vehicle steering systems, including procedures for working with high pressure and high temperature steering systems

environmental requirements, including procedures for trapping, storing and disposing of power steering hydraulic fluid released from steering systems

operating principles of light vehicle steering systems and associated components, including:

manual steering systems

power steering systems

application, purpose and operation of the following components of light vehicle steering systems and components, including:

steering wheels, including clock springs

steering columns

manual steering systems, including:

rack and pinion

steering boxes

hydraulic power steering systems, including:

integral piston

power assisted rack and pinion

electronically controlled power steering systems

electric steering systems, including steering angle sensors

identification, function and basic operating principles of steering angles, including:

steering angles:

toe-in and toe-out

toe-out on turns

caster

camber

Ackermann principle of steering

steering axis inclination

diagnostic testing procedures for light vehicle steering systems, including:

operational testing

hydraulic steering system testing

electronic and load sensing steering testing

analysing component wear analysis

repair procedures for light vehicle steering systems, including procedures for removing, replacing and adjusting the systems

post-repair testing procedures for light vehicle steering systems.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the light vehicle steering systems that they have worked on, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

manufacturer steering system specifications

one light vehicle with faults in its power-assisted rack and pinion steering system

diagnostic equipment for light vehicle steering systems

tools, equipment and materials appropriate for diagnosing and repairing light vehicle steering systems.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1