# Assessment Requirements for AURMTA008 Apply aerodynamic and vehicle dynamic principles and effects to competition vehicles

Release: 1

Assessment Requirements for AURMTA008 Apply aerodynamic and vehicle dynamic principles and effects to competition vehicles

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standards defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

determine and supervise the configuration of the following settings on two different competition vehicles:

tyre settings

steering and suspension settings

aerodynamic device settings.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to applying aerodynamic and vehicle dynamic principles and effects to competition vehicles, including procedures for identifying hazards and managing risks

key requirements of controlling body rules, category rules and supplementary regulations relating to establishing settings

terminology, principles and effects of the physics involved in resistance and motion

aerodynamic and vehicle dynamic principles and their effects on competition vehicles, including:

tyre settings, including:

construction and compound

size, including stagger

pressures

loadings

temperatures

steering and suspension settings, including:

wheel rate and linkage or leverage ratios

spring rate and spring pre-load

vehicle weight, sprung and unsprung weight

anti-roll bar rate where fitted

steering angles, including toe in and out, camber and caster

roll centres

ride height

centre of gravity

aerodynamic device settings, including:

wing angles, height and location

body panels and fittings

ground effects and ride height

frontal areas

lift reduction and drag reduction

downforce generation

downforce and drag compromise

competition vehicle test methods and procedures, including procedures for record keeping

mathematical computations for determining optimum settings.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the competition vehicle settings they have configured, e.g. work order.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

motor sport workplace or simulated workplace

two different competition vehicles requiring configuration resetting

controlling body rules, category rules and supplementary regulations for a motor sport event

tools and equipment appropriate for adjusting and modifying tyres, steering and suspension, and aerodynamic device settings.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1