# Assessment Requirements for AURMTF002 Analyse and repair performance fuel injection systems

Release: 1

Assessment Requirements for AURMTF002 Analyse and repair performance fuel injection systems

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standards defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

analyse and repair faults in the performance of the fuel injection systems of three different competition vehicles, including:

two of the following faults:

rough running

under or over fuelling

contamination or leaks

one of the following faults:

fuel cell ventilation

pumps

pressure and/or flow regulation

conduct post-repair checks on the above fuel systems.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to analysing and repairing performance fuel injection systems, including procedures for working with petrol fuel systems

environmental requirements, including procedures for trapping, storing and disposing of petrol fuel released during repair or testing

types, function, operation and limitations of fuel injection systems and components, including:

properties of fuels used in the motor sport industry, including compatibility with fuel injection system components

fuel injection system components, including:

air intake systems

fuel delivery systems, including fuel rails

fuel filters

fuel pumps

fuel surge tanks

adjustable fuel pressure regulators

fuel injectors, including peak-and-hold and saturated injectors:

fuel rails

fuel filters

fuel pumps

fuel surge tanks

adjustable fuel pressure regulators

management systems, including:

pressure and temperature sensors

electronic control units

diagnostic testing procedures of performance fuel injection systems, including:

symptom and cause differentiation

mapping fuel delivery

fuel trim and live data

fuel pump pressure, volume and vacuum testing

testing for air leaks

scan tool and data gathering

cold-start enrichment testing

exhaust gas analysis

procedures for repair and improvement of performance fuel injection systems, including fuel system component removal, replacement or repair, and adjustment procedures

post-repair testing procedures of performance fuel injection systems.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the performance fuel injection systems they have analysed and repaired, e.g. work order.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

manufacturer specifications for vehicle performance fuel injection systems

three different competition vehicles with faults in their performance fuel injection systems

diagnostic equipment for performance fuel injection systems, including:

exhaust gas analyser

scan tool or data logger

tools, equipment and materials appropriate for analysing and repairing performance fuel injection systems.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1