# Assessment Requirements for AURPTE004 Overhaul outdoor power equipment engines

Release: 1

Assessment Requirements for AURPTE004 Overhaul outdoor power equipment engines

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standard defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

overhaul two different outdoor power equipment engines as follows:

one four-stroke spark ignition engine

one of the following:

two-stroke spark ignition engine

four-stroke compression ignition engine.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to overhauling engines and associated engine components, including procedures for:

operating specialised engine overhaul tools, equipment and machinery

using cleaning chemicals and toxic substances

environmental requirements, including procedures for trapping, storing and disposing of lubricants and fluids released from engines

types, characteristics and operating principles of outdoor power equipment engines and associated engine components

outdoor power equipment engine overhaul procedures, including:

methods for cleaning and preparing an engine for overhaul

engine dismantling procedures

engine component inspection, measuring and evaluation procedures, including methods for measuring and calculating tolerances and clearances

engine and engine component repair and adjustment procedures, including:

engine cylinder head and cylinder block

engine crankshaft and camshaft

engine cylinder and engine sleeve fitting, boring and honing

engine bearing tunnel and connecting rod repair

engine assembly and adjustment procedures for achieving component tolerances, including:

piston to connecting rod big-end alignment

big-end bearing to crankshaft journal clearance

big-end bearing crush with bearing blue

piston ring end gap, back clearance and side clearance

main bearing to crankshaft journal clearance

cylinder head, valves, guides and seats

camshafts and cam followers

camshaft end float

crankshaft end float

gear backlash

governors

oil pump sealing and pick-up oil piping

component assembly procedures and processes, including:

welsh plugs and oil gallery plugs

piston and connecting rod assemblies

big-end bearings

piston rings to pistons

main bearings and thrust washers

crankshafts, including protection measures for crankshaft journals, bearings, rings and bores

finished camshaft bearings

cylinder head and components

camshafts and cam followers

timing gears

oil pumps, oil squirters, oil splashers, and oil pump pick-ups

governors

balance shafts

fitting ancillary components, including covers and cowlings, seals, housings, crankshaft pulleys and flywheels

post-overhaul testing procedures for engines, including:

fuel system adjustments

governor speed adjustments

engine protection devices.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the outdoor power equipment engines that they have worked on, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive workplace or simulated workplace

workplace instructions

manufacturer outdoor power equipment engine specifications

two different outdoor power equipment engines requiring overhaul

tools, equipment and materials appropriate for overhauling and adjusting outdoor power equipment engines.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1