# Assessment Requirements for AURRTC001 Diagnose and repair marine exhaust and cooling systems

Release: 1

Assessment Requirements for AURRTC001 Diagnose and repair marine exhaust and cooling systems

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standards defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

diagnose and repair a fault in the fresh water cooling exhaust and cooling systems of two different vessels

diagnose and repair a fault in the raw water exhaust and cooling systems of two different vessels.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to diagnosing and repairing marine exhaust and cooling systems, including procedures for:

lifting and supporting exhaust and cooling system components

working with hot exhaust and cooling system components

environmental requirements, including procedures for trapping, storing and disposing of materials released from exhaust systems

application, purpose and operation of marine exhaust and cooling systems and components, including:

fresh water cooling systems, including:

circulation pumps

thermostats

heat exchangers

exhaust manifolds

turbochargers

dry exhausts

raw water cooling systems, including:

pumps, including camshaft, belt, shaft and gear driven

relief valves

engine oil coolers

transmission oil coolers

sacrificial anodes

intercoolers

seawater strainers

exhaust manifolds

exhaust elbows and risers

exhaust tubes and hoses

wet exhausts

mufflers and waterlocks

siphon breaks

exhaust outlet flappers

diagnostic testing procedures for exhaust and cooling systems, including:

manometer measurement

turbocharger boost measurement

raw water system pressure and flow measurement

fresh water cooling system pressure testing

infra-red temperature measurement

dismantling and repair procedures for exhaust and cooling systems, including procedures for inspecting and evaluating the following components:

circulation pumps

thermostats

heat exchangers

turbochargers and intercoolers

raw water pumps, including camshaft, belt, shaft and gear driven

relief valves

sacrificial anodes

seawater strainers

exhaust manifolds

exhaust elbows and risers

exhaust tubes and hoses

mufflers and waterlocks

siphon breaks

exhaust outlet flappers

post-repair testing procedures for exhaust and cooling systems, including water testing procedures while under load.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the marine exhaust and cooling systems that they have worked on, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

marine repair workplace or simulated workplace

workplace instructions

manufacturer exhaust and cooling system specifications

two different marine vessels with faults in their fresh water cooling exhaust and cooling systems

two different marine vessels with faults in their raw water exhaust and cooling systems

diagnostic equipment for marine exhaust and cooling systems

tools, equipment and materials appropriate for dismantling, testing, repairing and reassembling marine exhaust and cooling systems.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1