# Assessment Requirements for AURRTE008 Install marine engines, controls and instruments

Release: 1

Assessment Requirements for AURRTE008 Install marine engines, controls and instruments

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standards defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

install and test prior to placing in service:

three different marine engines

the marine engine controls of the above engines

the marine instruments of the above engines.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to installing marine engines, controls and instrumentation, including lifting and supporting procedures for marine engines

environmental requirements, including procedures for trapping, storing and disposing of fluids released from engines

construction and operating principles of inboard and outboard engines, controls and instruments

procedures for installing inboard engines, controls and instruments into vessels, including:

assessing engine bed dimensions, strength and material to suit engine size and power

measuring engine position and determining propeller or input shaft alignment

evaluating transom to ensure it suits engine manufacturer requirements, including:

measuring transom thickness, parallelism, angle and surface flatness

assessing transom condition

cutting and sealing transom cut-out to suit stern drive application

sealing transom plate to transom

positioning, aligning and fastening front engine mounts

lifting, supporting and manoeuvring engine

checking and adjusting final alignment of engine

procedures for installing outboard engine, controls and instruments to vessels, including:

ensuring engine meets vessel specifications

determining engine position according to centre line and height

sealing transoms

procedures for testing the installation of inboard and outboard engines, controls and instruments in vessels, including checking for water leaks, fuel and oil leaks

commissioning procedures for inboard and outboard engines, controls and instruments, including testing vessels under normal operating conditions.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the engines, controls and instruments that they have installed in vessels, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

marine repair workplace or simulated workplace

workplace instructions

manufacturer engine, controls and instrument specifications

three different vessels requiring installation of engines, controls and instruments

tools, equipment and materials appropriate for installing marine engines, controls and instruments.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1