# Assessment Requirements for AURRTE012 Analyse and evaluate faults in light marine engine and powerhead systems

Release: 1

Assessment Requirements for AURRTE012 Analyse and evaluate faults in light marine engine and powerhead systems

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standards defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

analyse and evaluate a fault in the engine and power head systems of three different light marine vessels, including:

a fault in one inboard engine system

a fault in one outboard engine system.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to analysing and evaluating faults in light marine engine and power head systems, including procedures for:

lifting and supporting engines

running engines, including working with:

exhaust fumes

rotating shafts

environmental requirements, including procedures for trapping, storing and disposing of fluids released from engine and power head systems

principles and processes involved in planning and implementing analysis and evaluation of engine and power head systems

design and planning of diagnostic procedures of light marine engine and powerhead systems

design, manufacture, operation and limitations of light marine engine and power head systems and components, including:

two-stroke and four-stroke petrol engines

diesel engines

vessel systems and their impact on engine operation, including:

ignition systems

intake and exhaust systems

lubrication systems

cooling systems

engine mounting systems

testing procedures for light marine engine and powerhead systems, including:

non-destructive testing (NDT)

engine performance and maximum speed

full load water testing

types, functions, operation and limitations of diagnostic testing equipment required to analyse and evaluate faults in light marine engine and power head systems

procedures for documenting and reporting the analysis and evaluation process

requirements of Australian Design Rules (ADRs) relating to light marine engine and power head systems.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the light marine engine and power head systems that they have worked on, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

marine repair workplace or simulated workplace

workplace instructions

manufacturer engine and power head system specifications

three different light marine vessels with faults in the light marine engine and power head systems specified in the performance evidence

diagnostic equipment for light marine engine and power head systems

tools, equipment and materials appropriate for analysing and evaluating light marine engine and power head systems.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1