# AURRTQ002 Inspect and service marine jet drive propulsion systems

Release: 1

AURRTQ002 Inspect and service marine jet drive propulsion systems

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Application

This unit describes the performance outcomes required to inspect and service jet drive propulsion systems on vessels. It involves preparing for the task, inspecting the system, reporting the inspection findings, servicing and adjusting the system, and completing workplace processes and documentation.

It applies to those working in the marine service and repair industry.

No licensing, legislative, regulatory or certification requirements apply to this unit at the time of publication.

# Competency Field

Marine

# Unit Sector

Technical - Driveline and Final Drives

# Elements and Performance Criteria

| Elements Elements describe the essential outcomes. | Performance Criteria Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold and italicised text is used, further information is detailed in the range of conditions section. |
| --- | --- |
| 1. Prepare to inspect and service jet drive propulsion system | 1.1 Job requirements are determined from workplace instructions 1.2 Servicing information is sourced and interpreted 1.3 Hazards associated with the work are identified and risks are managed 1.4 Tools, equipment and materials are selected and checked for serviceability |
| 2. Inspect propulsion system | 2.1 Inspection is carried out according to manufacturer specifications, workplace procedures, and safety and environmental requirements 2.2 Inspection results are compared with manufacturer specifications 2.3 Inspection findings are reported according to workplace procedures, including recommendations for necessary repairs or adjustments |
| 3. Service propulsion system | 3.1 Service and adjustments are carried out according to manufacturer specifications, workplace procedures, and safety and environmental requirements, and without causing damage to components or systems 3.2 Post-service testing is carried out according to workplace procedures |
| 4. Complete work processes | 4.1 Final inspection is made to ensure work is to workplace expectations and vessel is presented ready for use 4.2 Work area is cleaned, waste and non-recyclable materials are disposed of, and recyclable material is collected 4.3 Tools and equipment are checked and stored according to workplace procedures 4.4 Workplace documentation is processed according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance and are not explicit in the performance criteria.

| Skills | Description |
| --- | --- |
| Learning skills to: | locate appropriate sources of information efficiently. |
| Reading skills to: | interpret information from manufacturer and workshop literature when seeking service procedures and specifications relating to jet drive propulsion systems. |
| Writing skills to: | legibly and accurately fill out workplace documentation when reporting inspection findings, making recommendations, and recording parts and material used. |
| Oral communication skills to: | clarify instructions report inspection findings and make repair recommendations. |
| Numeracy skills to: | use basic mathematical operations, including addition and subtraction, to calculate liquid volumes and service schedule intervals. |

# Range of Conditions

This section specifies work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. Bold italicised wording, if used in the performance criteria, is detailed below.

| Safety and environmental requirements must include: | work health and safety (WHS) and occupational health and safety (OHS) requirements, including procedures for identifying hazards and controlling risks associated with working with: exhaust fumes rotating components moving components that have the ability to crush environmental requirements, including procedures for trapping, storing and disposing of fluids released from propulsion systems. |
| --- | --- |

# Unit Mapping Information

Equivalent to AURRTQ2002 Service jet drive propulsion systems

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1