# Assessment Requirements for AURTTA027 Carry out basic vehicle servicing operations

Release: 1

Assessment Requirements for AURTTA027 Carry out basic vehicle servicing operations

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standards defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

correctly perform basic servicing operations on a minimum of two different operational vehicles.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

key aspects of work health and safety (WHS), occupational health and safety (OHS) and environmental requirements relating to basic vehicle servicing operations, including:

use of personal protective equipment, including safety glasses, ear protection and safety footwear

use of hand tools and lifting equipment

safety data sheets (SDS) and procedures for handling, storing and disposing of used oil, lubricants, coolants, transmission and brake fluids

reasons for servicing vehicles

basic vehicle servicing procedures, including:

servicing the engine:

checking for leaks, worn or loose fittings, cracks or other damage

changing the engine oil

changing the oil filter

servicing the drive belt:

checking for cracks, fraying, oil soaking and glazing at the belt to pulley contact area

adjusting the belt

servicing the cooling system:

checking for leaks, signs of corrosion, damaged or cracked hoses, worn or loose fittings

topping up the coolant

servicing transmission and final drive systems:

checking for leaks, worn or loose fittings, cracks or other damage

checking and topping up transmission and final drive lubricant level

topping up the clutch master cylinder fluid

servicing the tyres and suspension and steering system:

checking for leaks, splits in rubber boots, worn or loose fittings, and tyre wear

adjusting air pressure

lubricating ball joints

topping up power steering reservoir fluid

servicing the fuel system:

checking for leaks, worn or loose fittings, cracks or other damage

replacing air filters

servicing the electrical system:

checking the lighting system

checking and topping up the battery

servicing the brakes:

checking for leaks, wear, excessive heat damage, cracks or other damage

topping up the master cylinder fluid

other tasks, including checking:

windscreen wipers

windscreen washers (fluid level)

heating, ventilation and air-conditioning (HVAC) operation

body panels

exhaust system

mirrors

condition of foot pedal rubbers

hand and power tools and equipment used in vehicle servicing

work area clean-up and maintenance requirements.

# Assessment Conditions

Assessors must satisfy SNR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the vehicles that they have serviced, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive workplace or simulated workplace

personal protective equipment appropriate to the workplace

two vehicles for servicing

automotive hand and power tools and lifting and supporting equipment

automotive fluids and oils, including:

engine oil

transmission oil

brake fluid.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1