# AURTTB004 Inspect and service air braking systems

Release: 1

AURTTB004 Inspect and service air braking systems

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Application

This unit describes the performance outcomes required to inspect and service air braking systems according to manufacturer specifications. It involves preparing for the task, inspecting the system, reporting the inspection findings, servicing and adjusting the system, and completing workplace processes and documentation.

It applies to those working in the automotive service and repair industry. The air braking systems include those of agricultural machinery, heavy commercial vehicles or mobile plant machinery.

No licensing, legislative, regulatory or certification requirements apply to this unit at the time of publication.

# Competency Field

Mechanical Miscellaneous

# Unit Sector

Technical - Brakes

# Elements and Performance Criteria

| Elements Elements describe the essential outcomes. | Performance Criteria Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold and italicised text is used, further information is detailed in the range of conditions section. |
| --- | --- |
| 1. Prepare to inspect and service air braking system | 1.1 Job requirements are determined from workplace instructions 1.2 Servicing information is sourced and interpreted 1.3 Hazards associated with the work are identified and risks are managed 1.4 Tools, equipment and materials are selected and checked for serviceability |
| 2. Inspect air braking system | 2.1 Inspection is carried out according to manufacturer specifications, workplace procedures, and safety and environmental requirements 2.2 Inspection results are compared with manufacturer specifications 2.3 Inspection findings are reported according to workplace procedures, including recommendations for necessary repairs or adjustments |
| 3. Service air braking system | 3.1 Service and adjustments are carried out according to manufacturer specifications, workplace procedures, and safety and environmental requirements, and without causing damage to components or systems 3.2 Post-service testing is carried out according to workplace procedures |
| 4. Complete work processes | 4.1 Final inspection is made to ensure work is to workplace expectations and vehicle is presented ready for use 4.2 Work area is cleaned, waste and non-recyclable materials are disposed of, and recyclable material is collected 4.3 Tools and equipment are checked and stored according to workplace procedures 4.4 Workplace documentation is processed according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance and are not explicit in the performance criteria.

| Skills | Description |
| --- | --- |
| Learning skills to: | locate appropriate sources of information efficiently. |
| Reading skills to: | interpret information from manufacturer and workshop literature when seeking inspection and service procedures and specifications relating to air braking systems. |
| Writing skills to: | legibly and accurately fill out workplace documentation when reporting inspection findings, making recommendations, and recording parts and material used. |
| Oral communication skills to: | clarify instructions report inspection findings and make repair recommendations. |
| Numeracy skills to: | measure brake components and use basic mathematical operations, including addition and subtraction, to calculate distances, tolerances and deviations from manufacturer specifications interpret measuring equipment scales. |
| Technology skills to: | use precision measuring equipment, such as vernier calipers. |

# Range of Conditions

This section specifies work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. Bold italicised wording, if used in the performance criteria, is detailed below.

| Safety and environmental requirements must include: | work health and safety (WHS) and occupational health and safety (OHS) requirements, including procedures for: managing and controlling brake dust working with stored energy environmental requirements, including procedures for trapping, storing and disposing of brake dust released from air braking systems. |
| --- | --- |
| Components must include: | air compressor, filters, air drier, receivers and tanks drive belts spring brake chambers, hoses and adjusters discs, pads, drums and linings brake valves brake lights trailer brake valves. |
| Post-service testing must include: | static and dynamic testing of braking system. |

# Unit Mapping Information

Equivalent to AURTTB2004 Inspect and service air braking systems

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Assessment Requirements for AURTTB004 Inspect and service air braking systems

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standards defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

inspect and service the air braking systems of two different vehicles or machinery, including:

one air disc braking system

one air drum braking system.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to inspecting and servicing air braking systems, including procedures for:

managing and controlling brake dust

working with stored energy

environmental requirements, including procedures for trapping, storing and disposing of brake dust released from air braking systems

identification and function of major air braking components, including:

air compressor, filters, air drier, receivers and tanks

drive belts

spring brake chambers, hoses and adjusters

discs, pads, drums and linings

brake valves

brake lights

trailer brake valves

basic operation of air braking systems, including:

mechanical

pneumatic

inspection procedures for air braking systems, including:

component wear analysis

air system assessment

service and adjustment procedures for air braking systems, including:

brake adjustment

parking and emergency brake adjustment

post-service testing procedures for air braking systems, including static and dynamic testing.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the air braking systems that they have worked on, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

manufacturer air braking system specifications

two different vehicles or machinery with air disc and air drum braking systems requiring servicing

tools, equipment and materials appropriate for inspecting and servicing air braking systems.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1