# Assessment Requirements for AURTTE008 Dismantle and assemble multi-cylinder four-stroke petrol engines

Release: 1

Assessment Requirements for AURTTE008 Dismantle and assemble multi-cylinder four-stroke petrol engines

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standards defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

dismantle and assemble at least two different multi-cylinder four-stroke petrol engines.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

key aspects of work health and safety (WHS), occupational health and safety (OHS) and environmental requirements, including:

use of safety glasses, ear protection and safety footwear

use of engine dismantling and re-assembly equipment

collection and disposal of used engine lubricants and coolant

types of engines and engine operating principles for:

four and two-stroke engines

single and multi-cylinder configuration, including straight, vee and horizontally-opposed engine configurations

construction and operating principles of multi-cylinder four-stroke engine and functions of their major components

functions of the lubricating system

components of the lubricating system and their function, including:

sump

oil pumps

oil filters

oil galleries

functions of the cooling system

components of the cooling system and their function, including:

engine cooling galleries

radiators and hoses

thermostats

types and applications of:

seals

gaskets

bearings

dismantling procedures for engines, including:

bolt loosening sequences

procedures for noting component positions and order

measurement and calculation methods for:

piston to bore and connecting rod bearing clearance

swept volume

clearance volume

compression ratio and engine capacity

reassembly procedures for engines, including:

bolt tightening sequences and torque requirements

piston ring compression procedures

procedures for fitting pistons in bores in pairs

work area clean-up and maintenance requirements.

# Assessment Conditions

Assessors must satisfy SNR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the engines they have dismantled and re-assembled, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive workplace or simulated workplace

personal protective equipment appropriate to the workplace

two different multi-cylinder four-stroke petrol engines

tools and measuring instruments, including lifting equipment.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1