# Assessment Requirements for AURTTL007 Inspect and service LPG fuel systems

Release: 1

Assessment Requirements for AURTTL007 Inspect and service LPG fuel systems

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standards defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

inspect and service the following liquefied petroleum gas (LPG) fuel systems:

two different light vehicle LPG fuel systems

or

two different heavy commercial vehicle LPG fuel systems or

two different forklift LPG fuel systems.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to inspecting and servicing LPG fuel systems, including procedures for:

working with heavier-than-air flammable gas

avoiding cold burns

storing LPG

environmental requirements, including procedures for preventing loss of gas to atmosphere

licensing requirements for working on LPG systems

identification and function of LPG fuel system components

basic operation of LPG fuel systems

requirements of AS/NZS 1425 LP Gas fuel systems for vehicle engines

inspection procedures for LPG fuel systems according to relevant Australian standards, including:

leakage check

fuel container life

container corrosion and attachment checks

accuracy of the automatic fuel limiter

operation of fuel containment system, including testing automatic fuel shut-off device

fuel control systems, including automatic fuel shut-off device and fuel selector

manual valves

compartment or sub-compartment

filler connection

hydrostatic relief valve

safety valve system

liquids

ground clearances

vehicle identification, including identification markers

service and adjustment procedures for LPG fuel systems, including:

draining oil from converters

checking compliance of fuel system emissions

post-service testing procedures for LPG fuel systems.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the vehicle LPG fuel systems that they have worked on, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

AS/NZS 1425 LP Gas fuel systems for vehicle engines

manufacturer servicing schedules and vehicle LPG fuel system specifications

vehicles and machinery specified in the performance evidence with operating LPG fuel systems requiring servicing

tools, equipment and materials appropriate for inspecting and servicing LPG fuel systems.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1