# Assessment Requirements for AURTTM004 Assemble engine blocks and sub-assemblies

Release: 1

Assessment Requirements for AURTTM004 Assemble engine blocks and sub-assemblies

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standards defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

assemble two different engine blocks, one of which must be a V configuration engine block, and the sub-assemblies of those engine blocks, in which the work must involve:

a multi-cylinder diesel engine

a multi-cylinder petrol engine.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to assembling engine blocks and sub-assemblies, including procedures for:

selecting and using personal protective equipment (PPE) for handling engine blocks and sub-assemblies and using chemical cleaning agents

correctly operating specialised equipment, including heating torches, ovens and presses

handling freezing substances, including liquid nitrogen

manual handling techniques, including:

using machinery for lifting engine blocks and engine components

using slings, chains and other lifting equipment according to safe work practices

environmental requirements, including procedures for trapping, storing and disposing of cleaning fluids released during the assembly process

procedures for clearance testing and adjusting or machining components, including:

piston to connecting rod big end alignment

big end and main bearing crush with bearing blue

piston ring end gap, back clearance and side clearance

bearing to crankshaft clearances with plastigage and measuring and calculating clearances

semi-flanged thrust bearings

finished camshaft bearings and measuring clearances

camshafts and cam followers

camshaft end float

crankshaft end float

gear backlash

oil pump sealing and pick-up oil piping

component assembly procedures and processes, including:

pre-assembly wash procedures for components

welsh plugs and oil gallery plugs

piston and connecting rod assemblies

big end bearings

piston rings to pistons, including procedures for using ring compressors

main bearings and thrust washers

semi-flanged thrust bearings

crankshafts, including protection measures for crankshaft journals, bearings, rings and bores

finished camshaft bearings

camshafts and cam followers

timing gears

chains and tensioners

oil pumps, oil squirters and oil pump pick-ups

balance shafts

fitting ancillary components, including:

covers and seals

housings

diesel fuel injection pumps

oil coolers

crankshaft pulleys

flywheels

bell housings

requirements of Australian standards relevant to engine reconditioning, including:

AS 4182 Automotive repairs – Code of practice for reconditioning reciprocating spark ignition engines

AS 4427 Automotive repairs – Code of practice for reconditioning reciprocating compression ignition engines.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the engine block and sub-assemblies that they have assembled, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

engine specifications

AS 4182 Automotive repairs – Code of practice for reconditioning reciprocating spark ignition engines

AS 4427 Automotive repairs – Code of practice for reconditioning reciprocating compression ignition engines

two different multi-cylinder engine blocks, including:

in-line engine blocks

V configuration engine blocks

full gasket and seal set for the engine block

fixed and portable hand, air and power tools, lifting equipment, torque wrench and material suitable for assembling engine block

precision measuring equipment, including:

dial bore gauges

dial indicators

inside and outside metric and imperial micrometers.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1