# Assessment Requirements for AURTTM006 Perform advanced machining and blueprinting of engine components

Release: 1

Assessment Requirements for AURTTM006 Perform advanced machining and blueprinting of engine components

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standards defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

machine and blueprint two of the following different engine components:

engine with multiple overhead camshafts

V configuration engine with at least six cylinders

multi-cylinder diesel engine.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to performing advanced machining and blueprinting of engine components, including:

procedures for selecting and using personal protective equipment (PPE) for:

handling engine blocks and sub-assemblies

using chemical cleaning agents

operational risk assessment and treatments associated with:

electrical safety of machinery

machinery movement and operation

environmental requirements, including procedures for trapping, storing and disposing of cleaning and lubricating fluids released during process

procedures for preparing cylinder blocks and cylinder heads for blueprinting, including:

chemically cleaning engine components, including methods for avoiding damage or loss of components

chasing threads to remove debris with correct grade tap

hardness testing components

pressure testing cylinder heads

checking cylinder heads for bend on both sides

checking cylinder heads for minimum thickness

checking valve guides, valve stems, collets and valve springs for wear

testing engine block, including sonic testing and crack testing

dummy assembling engines, including reasons for dummy assembling engines

procedures for blueprinting cylinder blocks, including:

line boring main tunnels to produce datum

fitting bearings and checking crush

measuring main bearing tunnel with bearings fitted

grinding crankshaft to suit bearings and to achieve desired clearance and phasing consistency

sleeving lifter bores to centre line of camshaft and checking lifter to bore clearance and alignment to camshaft

fitting camshaft bearings and checking running clearances

squaring up deck to crankshaft tunnel to ensure crankshaft is parallel to block

fitting torque plate to bore and hone

honing cylinders to desired surface finish, matching individual pistons to respective bores

setting ring end gap

achieving consistent piston height for desired compression ratio and checking rotational clearance

procedures for blueprinting connecting rods, including:

shot peening connecting rods

preparing rod for re-sizing, including removing and locating dowels as required

selecting bolts

tensioning and re-sizing tunnel, maintaining centre-to-centre height

checking little end running clearance

removing bushes and checking parent bore of rod and bush fitment

pin fitting rod, maintaining centre-to-centre height and running clearance

checking alignment of rod

fitting up bearings, checking crush and running clearance

resizing connecting rods and fitting new bolts to ensure centre-to-centre length and alignment is correct

procedures to blueprint cylinder heads, including:

methods for achieving optimum airflow, including limitations of cylinder head modifications

machining for larger valves, including identifying machining shifts in castings

operating flow bench

porting for improved air flow and to balance combustion chambers

calculating capacity of individual combustion chambers and machining combustion chambers to achieve consistency

calculating compression ratio

selecting camshaft

setting valve heights

calculating installed height of valve springs and seat pressure

calculating valve spring open height and pressure

calculating valve spring solid height and coil bind clearance

calculating spring retainer to stem seal clearance

dialling camshaft

machining pistons to achieve desired compression ratio

setting rocker geometry, including length and sweep

requirements of Australian standards relevant to engine reconditioning, including:

AS 4182 Automotive repairs – Code of practice for reconditioning reciprocating spark ignition engines

AS 4427 Automotive repairs – Code of practice for reconditioning reciprocating compression ignition engines

post-grinding operations, including rust protection of machined surfaces.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the engine components that they have machined and blueprinted, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

manufacturer engine specifications

AS 4182 Automotive repairs – Code of practice for reconditioning reciprocating spark ignition engines

AS 4427 Automotive repairs – Code of practice for reconditioning reciprocating compression ignition engines

two different multi-cylinder engines as specified in the performance evidence requiring machining and blueprinting

precision measuring equipment, including:

dial bore gauges

dial indicators

inside and outside metric and imperial micrometers

inside and outside spring calipers

fixed and portable hand, air and power tools

lifting equipment

material suitable for machining and blueprinting the engine components.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1