# Assessment Requirements for AURTTM009 Fit sleeves and bore and hone engine cylinders

Release: 1

Assessment Requirements for AURTTM009 Fit sleeves and bore and hone engine cylinders

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standards defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

work on three different engine blocks as follows:

bore one engine block to accept parallel liners with correct interference fit

bore one engine block to accept flanged liners with correct interference fit

bore and hone the cylinders of a different multi-cylinder engine block to achieve correct piston to bore clearance.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to fitting sleeves and boring and honing engine cylinders, including:

procedures for selecting and using personal protective equipment (PPE) for:

handling engine cylinder blocks

using boring and honing machines

using chemical cleaning and lubricating agents

safe operating procedures for boring and honing machinery, including:

electrical safety of boring and honing machinery

boring and honing machinery movement and operation

environmental requirements, including procedures for trapping, storing and disposing of cleaning and lubricating fluids released during the machining process

types and application of sleeves, including:

dry sleeves, including:

interference fit parallel sleeve

interference fit flanged sleeves

chrome type finished-to-size dry flanged sleeves

wet sleeves, including:

flanged

stepped

sleeve fitting procedures, including:

interference sleeves, including fitting parallel sleeves to step in parent bore

wet sleeves

checking sleeve height

types, characteristics and limitations of honing and boring machines, including:

types and grades of boring tools

types and grades of honing stones

hand-held and machine-operated hones

tool sharpening methods, including:

maintaining sharpness of tool throughout boring operations

compensation methods for tool wear throughout boring operations

boring methods and procedures, including:

determining required size of bore, including honing allowance

preparing engine cylinder block for boring, and setting engine cylinder block into boring machine, including jigs, rails and clamps

setting boring tool to correct diameter, locking boring tool, and setting limit stop

reasons for taking a test cut

speed and feed rates of boring tool for differing materials, including alloysil or nicolsil, and for bore diameters

methods for setting speed and feed rate of boring tools

honing methods and procedures, including:

cleaning engine cylinder block before and after honing

preparing portable and stationary honing machines, setting engine cylinder block into honing machine, and setting hone stroke

setting honing speeds and feed rates to achieve required finish

methods of measuring bores for taper, ovality and barrelling

types and application of cleaning, lubricating and protective agents, including:

material suitability

application of lubricating agents for different speeds and feed rates of boring machines and honing machines

hazards associated with chemical cleaning and lubricating agents

requirements of Australian standards relevant to engine reconditioning, including:

AS 4182 Automotive repairs – Code of practice for reconditioning reciprocating spark ignition engines

AS 4427 Automotive repairs – Code of practice for reconditioning reciprocating compression ignition engines

post-repair procedures, including rust protection.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the engine cylinders that they have worked on, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

manufacturer engine specifications

AS 4182 Automotive repairs – Code of practice for reconditioning reciprocating spark ignition engines

AS 4427 Automotive repairs – Code of practice for reconditioning reciprocating compression ignition engines

three different multi-cylinder engine blocks as specified in the performance evidence

engine boring and honing machines

precision measuring equipment, including:

dial bore gauges

dial indicator gauges

inside and outside metric and imperial micrometers.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1