# AURTTX106 Diagnose and repair hydrostatic transmissions

Release: 1

AURTTX106 Diagnose and repair hydrostatic transmissions

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with AUR Automotive Retail, Service and Repair Training Package Version 6.0 |

# Application

This unit describes the skills and knowledge required to diagnose and repair faults in hydrostatic transmissions and their associated components. The unit involves preparing for the task, sourcing a diagnostic testing strategy, diagnosing the cause of the fault, carrying out the repair, performing post-repair testing, and completing workplace processes and documentation.

The unit applies to those who work in the automotive service and repair industry. The hydrostatic transmissions include those in agricultural machinery, mobile plant machinery or outdoor power equipment.

No licensing, legislative, regulatory or certification requirements apply to this unit at the time of publication.

# Unit Sector

Mechanical Miscellaneous Technical – Transmissions

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare to diagnose and repair heavy vehicle hydrostatic transmission | 1.1 Identify job requirements from workplace instructions 1.2 Identify required information for diagnosis activity 1.3 Analyse diagnostic options and source testing strategy to identify cause of fault using workplace and manufacturer procedures 1.4 Identify hazards and environmental issues associated with diagnose and repair activity, assess potential risks and implement control measures in line with workplace policies and procedures 1.5 Identify tools and equipment required for testing strategy and establish serviceability according to workplace procedures |
| 2. Diagnose heavy vehicle hydrostatic transmission | 2.1 Implement diagnostic tests set out in testing strategy according to manufacturer and workplace procedures, and workplace health and safety requirements 2.2 Identify cause of fault through analysis of diagnostic test results 2.3 Confirm and report cause of fault according to workplace procedures 2.4 Develop and report recommendations for necessary repairs according to workplace procedures |
| 3. Repair heavy vehicle hydrostatic transmission | 3.1 Identify required information for repair activity 3.2 Identify tools, equipment and materials required for repair activity and establish serviceability according to workplace procedures 3.3 Carry out repairs according to workplace and manufacturer procedures, manufacturer specifications, workplace health and safety and environmental requirements 3.4 Carry out post-repair testing according to workplace procedures, workplace health and safety and environmental requirements |
| 4. Complete work processes | 4.1 Conduct final inspection according to workplace procedures and confirm vehicle is ready for use 4.2 Clear work area and dispose of or recycle materials according to workplace procedures 4.3 Complete documentation according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance but not explicit in the performance criteria.

| SKILL | DESCRIPTION |
| --- | --- |
| Learning | Locates required sources of information efficiently Develops a sequenced plan for a specific task |
| Reading | Organises and interprets technical information from workplace procedures, manufacturer procedures and manufacturer specifications |
| Oral communication | Clarifies instructions Obtains information from customers and supervisors |
| Numeracy | Measures hydrostatic transmission components Uses mathematical operations Calculates distances, areas and tolerances Calculates deviations from manufacturer specifications |
| Planning and organising | Plans own work requirements Prioritises actions to achieve required outcomes Ensures tasks are completed within workplace timeframes |
| Technology | Uses specialised diagnostic equipment |

# Unit Mapping Information

Supersedes and is equivalent to AURTTX006 Diagnose and repair hydrostatic transmissions.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Assessment Requirements for AURTTX106 Diagnose and repair hydrostatic transmissions

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with AUR Automotive Retail, Service and Repair Training Package Version 6.0 |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

diagnose and repair a fault in the hydrostatic transmission systems of two different vehicles or machinery, including in three of the following components:

charge pump

main pump

drive motor

hydraulic lines

heat exchanger

control valve.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

methods to locate and interpret information required to diagnose and repair hydrostatic transmissions, including:

information provided by customers and supervisors

manufacturer specifications and procedures or equivalent documentation

workplace procedures required to diagnose and repair hydrostatic transmissions, including:

establishing serviceability of tools and equipment

documentation procedures

housekeeping procedures, including:

examination of tools and equipment

storage of equipment

identification, tagging and isolation of faulty equipment

disposal of excess materials

recycling procedures

workplace health and safety (WHS) requirements relating to diagnosing and repairing hydrostatic transmissions, including procedures for:

working with high pressure and stored energy hazards

isolating and stabilising machines

environmental requirements relating to diagnosing and repairing hydrostatic transmissions, including procedures for trapping, storing and disposing of fluids released from hydrostatic transmissions

operating principles of hydrostatic transmissions and associated components, including:

closed loop

charge circuitry

flushing circuit

purpose and operation of hydrostatic transmissions and components, including:

charge pump

main pump, including pilot control systems

drive motor, including control systems

filtering

hydraulic lines

heat exchanger

reservoir

diagnostic testing procedures for hydrostatic transmissions, including:

testing charging system

testing closed loop system

testing flushing system

measuring case drains

testing internal and external fluid leaks

checking abnormal noises and operation

using scan tools and multimeter

repair procedures for hydrostatic transmissions, including procedures for:

isolating faults

dismantling components and parts

inspecting and evaluating worn components

repairing and replacing parts

post-repair flow and pressure testing procedures for hydrostatic transmissions.

# Assessment Conditions

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the hydrostatic transmissions that they have worked on, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

manufacturer specifications for hydrostatic transmissions

two different vehicles or machinery with faults in the hydrostatic transmission components specified in the performance evidence

diagnostic equipment for hydrostatic transmissions

tools, equipment and materials appropriate for repairing and adjusting hydrostatic transmissions.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1