# AURVTG011 Install side vehicle windows

Release: 1

AURVTG011 Install side vehicle windows

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Application

This unit describes the performance outcomes required to fabricate templates for vehicle side windows, mark and cut panels, prepare cut edges, cut glass or perspex, and install the side windows. It involves preparing for the task, selecting and using specialist tools and equipment, selecting materials, checking material quality, sealing and installing side windows according to manufacturer specifications, cleaning and finishing the windows, and completing workplace processes and documentation.

It applies to those working in the automotive glazing service and repair industry.

No licensing, legislative, regulatory or certification requirements apply to this unit at the time of publication.

# Competency Field

Vehicle Body

# Unit Sector

Technical - Glazing

# Elements and Performance Criteria

| Elements Elements describe the essential outcomes. | Performance Criteria Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold and italicised text is used, further information is detailed in the range of conditions section. |
| --- | --- |
| 1. Prepare to install side window | 1.1 Job requirements are determined from workplace instructions 1.2 Information is accessed and interpreted from manufacturer specifications, workplace procedures, and sealant safety data sheets (SDS) 1.3 Work is planned to minimise waste and use time efficiently 1.4 Vehicle is inspected to ensure there is no prior damage and that all related accessories are operating correctly 1.5 Materials required to install side window are selected and checked for quality 1.6 Hazards associated with the work are identified and risks are managed 1.7 Tools and equipment for making opening and installing side window are selected and checked for serviceability 1.8 Vehicle is protected and safely located according to workplace procedures |
| 2. Fabricate templates | 2.1 Accurate measurements of shape and size of side window are taken in order to prepare templates 2.2 Suitable materials and equipment are used to fabricate templates 2.3 Templates are completed according to workplace procedures and without causing damage to vehicle or components |
| 3. Mark out and cut panel for side window | 3.1 Template is used to mark out areas on vehicle to be cut 3.2 Window opening is cut in panel using approved methods and equipment according to workplace procedures and safety and environmental requirements 3.3 Panel edges and trim are prepared for installation of side window 3.4 Panels and trim are reinforced as required to comply with vehicle manufacturer specifications and without causing damage to vehicle or components |
| 4. Install side window | 4.1 Material to be installed in vehicle side opening is selected and prepared 4.2 Side window is installed to manufacturer specifications using either rubber or direct glazing according to safety and environmental requirements 4.3 Window in panel opening is secured using approved materials |
| 5. Complete work processes | 5.1 Final inspection is made to ensure work meets workplace expectations, related accessories are operating correctly, customer is informed of time to wait before driving vehicle, and vehicle is presented ready for use 5.2 Work area is cleaned, waste and non-recyclable materials are disposed of, and recyclable material is collected and stored 5.3 Tools and equipment are checked, reported if faulty, and stored according to workplace procedures 5.4 Workplace documentation is processed according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance and are not explicit in the performance criteria.

| Skills | Description |
| --- | --- |
| Learning skills to: | locate appropriate sources of information efficiently. |
| Writing skills to: | legibly and accurately fill out workplace documentation. |
| Oral communication skills to: | clarify instructions and procedures clearly report issues or outcomes. |
| Numeracy skills to: | use basic mathematical operations, including addition and subtraction, to: calculate template and panel measurements cut opening. |
| Planning and organising skills to: | plan own work requirements and prioritise actions to achieve required outcomes and ensure tasks are completed within workplace timeframes. |
| Problem solving skills to: | identify quality issues, reinforcement requirements and potential problems associated with cutting opening and installing side windows. |
| Technology skills to: | use specialist tools and measuring equipment. |

# Range of Conditions

This section specifies work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. Bold italicised wording, if used in the performance criteria, is detailed below.

| Materials must include: | side window panel material template materials trim sealants and adhesives cleaning agents. |
| --- | --- |
| Tools and equipment must include: | personal protective equipment (PPE) hand and power tools measuring equipment metal cutting equipment vehicle protection equipment templates. |
| Safety and environmental requirements must include: | work health and safety (WHS) and occupational health and safety (OHS) requirements, including procedures for: selecting and using PPE using tools and lifting equipment using SDS for sealants material handling and storage environmental requirements, including procedures for trapping, storing and disposing of waste materials and chemicals. |

# Unit Mapping Information

Equivalent to AURVTG3011 Install side windows

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1