# AURVTG101 Repair laminated glass windscreens

Release: 1

AURVTG101 Repair laminated glass windscreens

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with AUR Automotive Retail, Service and Repair Training Package Version 6.0 |

# Application

This unit describes the skills and knowledge required to repair chipped or cracked laminated glass windscreens. The unit involves preparing for the task, selecting and using specialist tools and equipment, cleaning and removing moisture from chipped or cracked laminated glass windscreens, injecting resin into chip or crack, finishing repair work to industry standard, and completing workplace processes and documentation.

The unit applies to those who work in the automotive glazing service and repair industry.

No licensing, legislative, regulatory or certification requirements apply to this unit at the time of publication.

# Unit Sector

Vehicle Body Technical – Glazing

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare to repair laminated glass windscreen | 1.1 Identify job requirements from workplace instructions 1.2 Check size of chip or crack against industry repair standards and identify compliance requirements 1.3 Identify required information for repair activity 1.4 Identify hazards associated with the work, assess potential risks and implement control measures in line with workplace policies and procedures 1.5 Identify tools, equipment and chemicals required for repair activity and establish serviceability according to workplace procedures |
| 2. Repair chipped and cracked laminated glass | 2.1 Carry out repair process for chipped or cracked glass according to workplace procedures, manufacturer specifications, industry standards and workplace health and safety and environmental requirements 2.2 Finish glass surface according to manufacturer specifications and safety data sheets (SDS) |
| 3. Complete work processes | 3.1 Conduct final inspection according to workplace procedures and confirm vehicle is ready for use 3.2 Clear work area and dispose of or recycle materials according to workplace procedures 3.3 Complete documentation according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance but not explicit in the performance criteria.

| SKILL | DESCRIPTION |
| --- | --- |
| Learning | Locates required sources of information efficiently |
| Reading | Organises and interprets technical information from workplace procedures, manufacturer procedures and manufacturer specifications |
| Oral communication | Clarifies instructions and procedures Reports quality issues and job outcomes clearly |
| Numeracy | Uses mathematical operations Calculates quantities of required materials |
| Planning and organising | Plans own work requirements Prioritises action to achieve required outcomes Ensures tasks are completed within workplace timeframes Carries out work without causing damage to vehicle or components |
| Technology | Uses specialised equipment and tools safely |

# Unit Mapping Information

Supersedes and is equivalent to AURVTG001 Repair laminated glass windscreens.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Assessment Requirements for AURVTG101 Repair laminated glass windscreens

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with AUR Automotive Retail, Service and Repair Training Package Version 6.0 |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

repair at least three different laminated glass windscreens, at least one of which must be chipped and at least one cracked.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

methods to locate and interpret information required to repair laminated glass windscreens, including:

AS/NZS 2366.1 Windscreen repairs - Repair procedures

AS/NZS 2366.2 Windscreen repairs - Repair systems

manufacturer specifications and procedures or equivalent documentation

safety data sheets (SDS) relating to repair chemicals and resins

workplace procedures required to repair laminated glass windscreens, including:

establishing the serviceability of tools and equipment

documentation procedures

housekeeping procedures, including:

examination of tools and equipment

storage of equipment

identification, tagging and isolation of faulty equipment

disposal of excess materials

recycling procedures

workplace health and safety (WHS) requirements relating to repairing laminated glass windscreens, including procedures for:

selecting and using personal protective equipment (PPE)

using glass repair systems

using safety data sheets (SDS) when working with chemicals and resins

material handling and storage

protection and safe location of vehicle

environmental requirements, including procedures for trapping, storing and disposing of waste chemicals, resins and materials

laminated glass repair systems and procedures for:

conducting inspection of vehicle for prior damage

identifying types and use of laminated glass repair resins

opening windscreen chip or crack to allow access for the repair resin

cleaning repair area and removing moisture from laminated glass

removing moisture from laminated glass

removing air from repair area using a vacuum tool

pressure injection of repair resin

ensuring capillary action

finishing glass surfaces

procedures for protecting vehicle systems and components during repair work

pre- and post-inspection procedures for laminated glass windscreen repairs.

# Assessment Conditions

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of task.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the laminated glass windscreens that they have repaired, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive glazing workplace, off-site workplace location or simulated workplace

workplace instructions

SDS for repair chemicals and resins

PPE required to repair laminated glass

AS/NZS 2366.1 Windscreen repairs - Repair procedures

AS/NZS 2366.2 Windscreen repairs - Repair systems

glass repair resins and materials

three different laminated glass windscreens as specified in the performance evidence requiring repair

vehicle protection equipment

cleaning products

tools, specialist equipment and materials appropriate for repairing laminated glass.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1