# Assessment Requirements for AURVTN031 Carry out vehicle sectional repairs

Release: 1

Assessment Requirements for AURVTN031 Carry out vehicle sectional repairs

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standard defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

complete sectional repairs on three different damaged vehicles, where each sectional repair must involve two of the following:

body pillar

body rail

sill panel

quarter panel.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to carrying out vehicle sectional repairs, including procedures for:

selecting and using personal protective equipment (PPE)

manually handling vehicle components

using tools and equipment

environmental requirements, including procedures for trapping, storing and disposing of waste materials from repair process

original equipment manufacturer (OEM) or authorised agency’s specifications and repair information, including:

OEM-approved bonding and riveting

sectional repair procedures

OEM-recommended procedures

removal of damaged components

repair, replacement and alignment of components

underbody measurements to OEM or authorised agency specifications

alignment or jigging systems

approved welding equipment and processes

pre- and post-repair measurements according to workplace procedures

procedures for identifying structural and non-structural components, including:

body pillars

body rails

sill panels

quarter panels

types of steels and their repair characteristics

techniques and procedures for carrying out sectional repairs, including:

types of repair tools and equipment and their operating procedures

use of structural foam

use of weld-through primer

alignment methods, techniques and procedures

approved welding and attachment procedures

bonding, riveting and sealant methods and procedures

procedures for protecting vehicle and components during repair work

procedures for final inspection of completed sectional repairs.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the vehicle sectional repairs they have completed, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

OEM or authorised agency’s specifications and procedures

vehicle specifications

PPE required to carry out sectional repairs

three different damaged vehicles requiring the sectional repairs specified in the performance evidence

replacement sectional components

tools, equipment and materials appropriate for carrying out vehicle sectional repairs.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1