# Assessment Requirements for AURVTN119 Repair vehicle structural damage by riveting

Release: 1

Assessment Requirements for AURVTN119 Repair vehicle structural damage by riveting

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with AUR Automotive Retail, Service and Repair Training Package Version 6.0 |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

complete repair and riveting activities on at least three different damaged vehicles.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

methods to locate and interpret information required to repair vehicle structural damage by riveting, including:

information provided by customer and supervisors

manufacturer specifications and procedures or equivalent documentation

workplace procedures required to repair vehicle structural damage by riveting, including:

establishing serviceability of tools and equipment

documentation procedures

housekeeping procedures, including:

examination of tools and equipment

storage of equipment

identification, tagging and isolation of faulty equipment

disposal of excess materials

recycling procedures

work health and safety (WHS) requirements relating to repairing vehicle structural damage by riveting, including procedures for:

selecting and using personal protective equipment (PPE)

manually handling vehicle panels

using tools and equipment

environmental requirements, including procedures for trapping, storing and disposing of waste materials

vehicle components for repair, including radiator support riveted assemblies and skirt and rail assemblies

original equipment manufacturer (OEM) or authorised agency’s approved riveting instructions, rivet specifications and quality standards

tools and equipment relating to repairing vehicle structural damage by riveting, including:

personal protective equipment (PPE)

hand and power tools and equipment

hydraulic pneumatic push/pull equipment

OEM or authorised agency’s approved riveting and measuring equipment

straightening systems

riveting procedures and techniques, including:

use of repair, riveting and measuring equipment

hydraulic straightening and realigning

body component measuring

panel replacement

procedures for final inspection of riveted structural repair.

# Assessment Conditions

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the vehicle structural damage they have repaired using riveting techniques, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

OEM or authorised agency’s approved repair and riveting procedures and specifications

PPE required to repair and rivet structurally damaged sections

materials required to rivet, repair and replace structurally damaged sections

three different structurally damaged vehicles requiring repair by riveting

tools, equipment and materials appropriate for repairing structural damage by riveting.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1