# AURVTN120 Remove and replace major welded panels on vehicles

Release: 1

AURVTN120 Remove and replace major welded panels on vehicles

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with AUR Automotive Retail, Service and Repair Training Package Version 6.0 |

# Application

This unit describes the skills and knowledge required to remove and replace vehicle welded panels and panel sections. This unit involves preparing for the task, selecting and using specialist tools and equipment, removing, repairing, aligning and welding major vehicle panels and sections. It also includes finishing repair to pre-paint condition and completing workplace processes and documentation.

The unit applies to those working in the automotive body repair industry.

No licensing, legislative, regulatory or certification requirements apply to this unit at the time of publication.

# Unit Sector

Vehicle Body Technical - Body

# Elements and Performance Criteria

| ELEMENTS | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare to replace major welded panels | 1.1 Determine job requirements from workplace instructions 1.2 Source vehicle specifications and original equipment manufacturer (OEM) or authorised agency’s recommended repair procedures and check against workplace instructions 1.3 Identify hazards associated with the work and manage risks 1.4 Select and check tools and equipment for serviceability 1.5 Plan work to identify methods, minimise waste, and use time efficiently |
| 2. Remove welded panels | 2.1 Remove major welded panels and sections according to OEM or authorised agency’s approved methods and safety and environmental requirements 2.2 Drill out spot welds using recommended drill bit and drill diameters 2.3 Seek assistance from authorised personnel where removal of welded panels and sections disturbs vehicle trim or electrical, mechanical or air conditioning systems |
| 3. Replace and align welded panels | 3.1 Replace, align and weld panel sections using approved OEM or authorised agency’s methods and procedures 3.2 Carry out replacement, alignment and welding of panel section to pre-paint condition |
| 4. Complete work processes | 4.1 Conduct final inspection according to workplace procedures and confirm vehicle is ready for use 4.2 Clear work area and dispose of or recycle materials according to workplace procedures 4.3 Complete documentation according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance but not explicit in the performance criteria.

| SKILL | DESCRIPTION |
| --- | --- |
| Learning | Locates required sources of information efficiently. |
| Numeracy | Measures using universal measuring system Uses basic mathematical operations, including addition and subtraction, to calculate: OEM or authorised agency repair measurements and tolerances material requirements Adjusts welder displays settings Sets alignment equipment. |
| Oral communication | Clarifies instructions and procedures Reports quality issues and job outcomes clearly. |
| Problem solving | Refers problems outside area of responsibility to required person. |
| Technology | Uses specialised tools and equipment. |

# Unit Mapping Information

Supersedes and is equivalent to AURVTN020 Remove and replace major welded panels on vehicles.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1