# Assessment Requirements for AURVTP013 Prepare vehicle substrates for refinishing

Release: 1

Assessment Requirements for AURVTP013 Prepare vehicle substrates for refinishing

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standard defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

prepare two different vehicle substrates for refinishing, in which the work must involve:

full steel vehicle body panel with surface rust

full aluminium vehicle body panel

stainless steel sheet measuring at least 150 mm x 150 mm or stainless steel body moulding

galvanised coated sheet measuring at least 150 mm x 150 mm b

zinc anneal sheet measuring at least 150 mm x 150 mm.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to preparing vehicle substrates for refinishing, including procedures for:

selecting and using personal protective equipment (PPE), including positive air feed masks

using technical data sheets (TDS) and safety data sheets (SDS) for primers and chemical cleaners

handling and storing paint materials

using tools and equipment

using approved drying areas

environmental requirements, including procedures for trapping, storing and disposing of waste materials

paint manufacturer specifications

types and uses of tools, equipment and materials, including:

primers and sealers

spray gun set-up, maintenance and application

cleaning agents

surface preparation methods and techniques for primers, sealers and surface repair:

masking off areas not to be coated

adhesion tests

primer mixing techniques

application methods for primers, fillers and sealers

sanding and blocking

feather edging procedures

procedures for protecting vehicle and components when preparing substrates

procedures for final inspection of prepared substrates.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of task.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the vehicle substrates that they have prepared for refinishing, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

TDS and SDS for primers and chemical cleaners

PPE required to prepare substrates for refinishing

materials required to prepare substrates for refinishing

vehicle substrates specified in the performance evidence requiring preparation for refinishing

tools, equipment and materials appropriate for preparing vehicle substrates for refinishing.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1