# AURVTP023 Apply clear over base two-pack refinishing materials to vehicle body components

Release: 1

AURVTP023 Apply clear over base two-pack refinishing materials to vehicle body components

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Application

This unit describes the performance outcomes required to mix and apply clear over base (COB) paint materials to vehicle body components. It involves preparing for the task, selecting and using specialist tools and equipment, identifying paint faults, preparing surface to be painted, selecting and applying COB two-pack refinishing materials according to manufacturer specifications, and completing workplace processes and documentation.

It applies to those working in the automotive paint refinishing repair industry.

No licensing, legislative, regulatory or certification requirements apply to this unit at the time of publication.

# Competency Field

Vehicle Body

# Unit Sector

Technical - Paint

# Elements and Performance Criteria

| Elements Elements describe the essential outcomes. | Performance Criteria Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold and italicised text is used, further information is detailed in the range of conditions section. |
| --- | --- |
| 1. Prepare to mix and apply COB two-pack refinishing materials to vehicle body components | 1.1 Job requirements are determined from workplace instructions 1.2 Vehicle paint specifications, technical data sheets (TDS) and paint safety data sheets (SDS) are accessed and interpreted 1.3 Refinishing materials are selected and inspected for quality 1.4 Hazards associated with the work are identified and risks are managed 1.5 Refinishing tools and equipment, including personal protective equipment (PPE), are selected and checked for serviceability 1.6 Work is planned to identify mixing and application methods for COB two-pack materials, minimise waste, prevent damage to vehicle, and use time efficiently |
| 2. Mix COB two-pack refinishing materials | 2.1 COB two-pack refinishing materials are calculated according to paint manufacturer specifications, formulas and codes 2.2 Codes and information relevant to job requirement are recorded 2.3 Materials are mixed according to workplace procedures and safety and environmental requirements, and without causing damage to components or systems |
| 3. Apply COB two-pack refinishing materials | 3.1 Refinishing materials are applied using manufacturer recommended method and intervals, and according to safety and environmental requirements 3.2 Refinishing materials are dried using approved methods and equipment to ensure painted surface meets specifications |
| 4. Rectify paint faults | 4.1 Paint faults are identified according to industry and workplace procedures 4.2 Rectification procedures are selected to suit fault or damage type 4.3 Materials to rectify paintwork are determined from paint manufacturer information 4.4 Paint surface faults are rectified using compounds, polishes and glazes to blend rectified areas with existing paintwork |
| 5. Complete work processes | 5.1 Final inspection is made to ensure work meets workplace expectations and component is presented ready for use 5.2 Work area is cleaned, waste and non-recyclable materials are disposed of, and recyclable material is collected and stored 5.3 Tools and equipment are checked, reported if faulty, and stored according to workplace procedures 5.4 Workplace documentation is processed according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance and are not explicit in the performance criteria.

| Skills | Description |
| --- | --- |
| Learning skills to: | locate appropriate sources of information efficiently. |
| Writing skills to: | legibly and accurately fill out workplace documentation. |
| Oral communication skills to: | clarify instructions and procedures. |
| Numeracy skills to: | interpret paint codes and formulas set and adjust spray painting air pressure and paint measuring system use basic mathematical operations, including addition, subtraction, multiplication and division to, determine ratios, measure and calculate quantities of paint and materials. |
| Planning and organising skills to: | plan own work requirements and prioritise actions to achieve required outcomes and ensure tasks are completed within workplace timeframes. |
| Technology skills to: | use specialist equipment, including: spray painting equipment electronic paint measuring and mixing system computerised drying equipment. |

# Range of Conditions

This section specifies work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. Bold italicised wording, if used in the performance criteria, is detailed below.

| Refinishing materials must include: | solid, metallic and pearl two-pack base coat clear two-pack acrylic enamel water-based paints hardener paint reducer cleaning solvents. |
| --- | --- |
| Tools and equipment must include: | spraying equipment paint tinting equipment approved spray booth or area drying equipment paint measuring system. |
| Safety and environmental requirements must include: | work health and safety (WHS) and occupational health and safety (OHS) requirements, including procedures for: selecting and using PPE using TDS and SDS for COB two-pack paint handling and storing paint materials using tools and equipment using approved drying area environmental requirements, including procedures for trapping, storing and disposing of waste materials. |
| Meeting specifications must include: | matching colour, texture, depth and gloss ensuring paint finish is contaminant-free ensuring paint finish blends into surrounding surfaces. |

# Unit Mapping Information

Equivalent to AURVTP3023 Mix and apply clear over-base refinishing materials in two-component systems

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Assessment Requirements for AURVTP023 Apply clear over base two-pack refinishing materials to vehicle body components

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standard defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

mix and apply clear over base (COB) two-pack refinishing materials to three different vehicles, including two of the following vehicle body components:

single vehicle panel

full vehicle side

vehicle front end or rear end.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to applying clear over base two-pack refinishing materials to vehicle body components, including procedures for:

selecting and using personal protective equipment (PPE)

using technical data sheets (TDS) and safety data sheets (SDS) for COB two-pack paint

handling and storing paint materials

using tools and equipment

using approved drying area

environmental requirements, including procedures for trapping, storing and disposing of waste materials

vehicle paint specifications

types and application of automotive paint, including correct use of positive air feed masks

mixing and application techniques for COB two-pack materials, including:

paint mixing techniques and systems

visual colour matching techniques and use of colour test cards

paint fault identification and rectification methods for COB two-pack refinishing materials

types and use of paint colour matching equipment, including:

mixing system set-up and operation

set-up, operation and maintenance of spray guns

paint drying equipment

paint cleaning materials and equipment

procedures for protecting vehicle and components when applying refinishing materials

procedures for final inspection of completed COB two-pack application.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of task.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the COB two-pack materials they have applied to vehicle body components, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

paint and vehicle manufacturer specifications and paint codes

TDS and SDS for COB two-pack paint

PPE, including positive air feed masks

COB refinishing materials

three different vehicles with body components specified in the performance evidence requiring painting with COB two-pack refinishing materials

tools, equipment and materials appropriate for applying COB two-pack refinishing materials.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1