# AURVTP123 Apply clear over base two-pack refinishing materials to vehicle body components

Release: 1

AURVTP123 Apply clear over base two-pack refinishing materials to vehicle body components

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with AUR Automotive Retail, Service and Repair Training Package Version 6.0 |

# Application

This unit describes the skills and knowledge required to mix and apply clear over base (COB) paint materials to vehicle body components. This unit involves preparing for the task, selecting and using specialist tools and equipment, identifying paint faults, preparing surface to be painted, selecting and applying COB two-pack refinishing materials according to manufacturer specifications, and completing workplace processes and documentation.

The unit applies to those working in the automotive refinishing industry.

No licensing, legislative, regulatory or certification requirements apply to this unit at the time of publication.

# Unit Sector

Vehicle Body Technical - Paint

# Elements and Performance Criteria

| ELEMENTS | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for work | 1.1 Identify job requirements from workplace instructions 1.2 Identify hazards associated with the work, assess potential risks and implement control measures in line with workplace policies and procedures 1.3 Identify tools, equipment and chemicals required for repair activity and establish serviceability according to workplace procedures |
| 2. Mix COB two-pack refinishing materials | 2.1 Calculate COB two-pack refinishing materials according to paint manufacturer specifications, formulas and codes 2.2 Record codes and information relevant to job requirement 2.3 Mix materials according to workplace procedures |
| 3. Apply COB two-pack refinishing material | 3.1 Apply refinishing materials using manufacturer recommended method and intervals, and according to workplace procedures 3.2 Dry refinishing materials and equipment according to workplace procedures and ensure painted surface meets specifications |
| 4. Rectify paint faults | 4.1 Identify paint faults according to industry and workplace procedures 4.2 Select rectification procedures according to fault or damage type 4.3 Identify required materials to rectify paintwork from paint manufacturer information 4.4 Rectify paint surface faults using compounds, polishes and glazes to blend rectified areas with existing paintwork |
| 5. Complete work processes | 5.1 Conduct final inspection according to workplace procedures and confirm vehicle is ready for use 5.2 Clear work area and dispose of or recycle materials according to workplace procedures 5.3 Complete documentation according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance but not explicit in the performance criteria.

| SKILL | DESCRIPTION |
| --- | --- |
| Learning | Locates required sources of information efficiently. |
| Reading | Interprets workplace instructions such as job cards Organises and interprets technical information from workplace procedures, manufacturer specifications, industry standards and paint technical data sheets (TDS) and safety data sheets (SDS). |
| Oral communication | Clarifies instructions and procedures. |
| Numeracy | Interprets paint codes and formulas Sets and adjusts spray painting air pressure and paint measuring system Uses basic mathematical operations, including addition, subtraction, multiplication and division to: determine ratios measure and calculate quantities of paint and materials. |
| Planning and organising | Plans own work requirements Prioritises actions to achieve required outcomes Ensures tasks are completed within workplace timeframes. |
| Technology | Uses specialist equipment. |

# Unit Mapping Information

Supersedes and is equivalent to AURVTP023 Apply clear over base two-pack refinishing materials to vehicle body components.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Assessment Requirements for AURVTP123 Apply clear over base two-pack refinishing materials to vehicle body components

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with AUR Automotive Retail, Service and Repair Training Package Version 6.0 |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

mix and apply clear over base (COB) two-pack refinishing materials to at least three different vehicles, at least two of which must include a different vehicle body component from among the following:

single vehicle panel

full vehicle side

vehicle front end or rear end.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

methods to locate and interpret information required to apply clear over base two-pack refinishing materials to vehicle body components, including:

vehicle paint specifications

workplace procedures required to apply clear over base two-pack refinishing materials to vehicle body components including:

establishing the serviceability of tools and equipment

documentation procedures

housekeeping procedures including:

examination of tools and equipment

storage of equipment

identification, tagging and isolation of faulty equipment

disposal of excess materials

recycling procedures

workplace health and safety (WHS) requirements relating to applying clear over base two-pack refinishing materials to vehicle body components, including procedures for:

selecting and using personal protective equipment (PPE)

using technical data sheets (TDS) and safety data sheets (SDS) for COB two-pack paint

handling and storing paint materials

using tools and equipment

using approved drying area

environmental requirements relating to applying clear over base two-pack refinishing materials to vehicle body components, including procedures for trapping, storing and disposing of waste materials

types and use of refinishing material, including:

solid, metallic and pearl two-pack base coat

clear two-pack acrylic enamel

water-based paints

hardener

paint reducer

cleaning solvents.

types and application of automotive paint, including correct use of positive air feed masks

mixing and application techniques for COB two-pack materials, including:

paint mixing techniques and systems

visual colour matching techniques and use of colour test cards

paint fault identification and rectification methods for COB two-pack refinishing materials

types and use of paint colour matching equipment, including:

mixing system set-up and operation

set-up, operation and maintenance of spray guns

paint drying equipment

paint cleaning materials and equipment

procedures for protecting vehicle and components when applying refinishing materials

procedures for final inspection of completed COB two-pack application.

# Assessment Conditions

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the COB two-pack materials they have applied to vehicle body components, e.g. repair orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

paint and vehicle manufacturer specifications and paint codes

TDS and SDS for COB two-pack paint

PPE, including positive air feed masks

COB refinishing materials

three different vehicles with body components specified in the performance evidence requiring painting with COB two-pack refinishing materials

tools, equipment and materials appropriate for applying COB two-pack refinishing materials.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1