# AURVTT013 Fabricate and fit loose and fitted covers to vehicle seats and components

Release: 1

AURVTT013 Fabricate and fit loose and fitted covers to vehicle seats and components

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Application

This unit describes the performance outcomes required to measure, mark out and cut materials in order to fabricate and fit covers to vehicle seats and components. It involves preparing for the task, selecting and using specialist tools and equipment, selecting materials and checking their quality, following manufacturer and customer instructions, inspecting finished work, and completing workplace processes and documentation.

It applies to those working in the automotive trimming industry.

No licensing, legislative, regulatory or certification requirements apply to this unit at the time of publication.

# Competency Field

Vehicle body

# Unit Sector

Technical - Trimming and Upholstery

# Elements and Performance Criteria

| Elements Elements describe the essential outcomes. | Performance Criteria Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold and italicised text is used, further information is detailed in the range of conditions section. |
| --- | --- |
| 1. Prepare to fabricate loose and fitted covers | 1.1 Job requirements are determined from workplace instructions 1.2 Materials are selected and inspected for quality and quantities are calculated 1.3 Hazards associated with the work are identified and risks are managed 1.4 Tools and equipment, including personal protective equipment (PPE), are selected for appropriate fabrication methods and checked for serviceability 1.5 Work area is prepared and work is planned to minimise waste and use time efficiently |
| 2. Measure, mark out and cut materials | 2.1 Templates and specialist tools and equipment are selected and used according to safety and environmental requirements 2.2 Materials are marked out and cut according to patterns, templates workplace instructions, manufacturer specifications and fitting requirements |
| 3. Fabricate, install and fasten covers | 3.1 Covers are fabricated and assembled according to job requirements and workplace instructions 3.2 Covers are installed and fastened without causing damage to vehicle or components 3.3 Fitted covers are checked for alignment and quality according to job requirements, workplace procedures and manufacturer specifications |
| 4. Complete work processes | 4.1 Final inspection is made to ensure work meets workplace expectations and component is presented ready for use 4.2 Work area is cleaned, waste and non-recyclable materials are disposed of, and recyclable material is collected and stored 4.3 Tools and equipment are checked, reported if faulty, and stored according to workplace procedures 4.4 Workplace documentation is processed according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance and are not explicit in the performance criteria.

| Skills | Description |
| --- | --- |
| Learning skills to: | locate appropriate sources of information efficiently. |
| Writing skills to: | legibly and accurately fill out workplace documentation. |
| Oral communication skills to: | clarify instructions and procedures clearly report quality issues and job outcomes. |
| Numeracy skills to: | use basic mathematical operations, including addition and subtraction, to calculate: fitting measurements quantities of required materials. |
| Planning and organising skills to: | plan own work requirements and prioritise actions to achieve required outcomes and ensure tasks are completed within workplace timeframes. |
| Technology skills to: | use specialist tools and equipment. |

# Range of Conditions

This section specifies work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. Bold italicised wording, if used in the performance criteria, is detailed below.

| Fabrication methods must include: | measuring, marking out and cutting machine sewing fastening, shaping and gluing assembling and fitting. |
| --- | --- |
| Safety and environmental requirements must include: | work health and safety (WHS) and occupational health and safety (OHS) requirements, including procedures for: selecting and using PPE using specialist tools and equipment manual handling vehicle components environmental requirements, including procedures for storing and disposing of waste materials. |

# Unit Mapping Information

Equivalent to AURVTT3013 Fabricate loose and fitted covers

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Assessment Requirements for AURVTT013 Fabricate and fit loose and fitted covers to vehicle seats and components

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | New unit of competency. |

# Performance Evidence

Before competency can be determined, individuals must demonstrate they can perform the following according to the standard defined in this unit’s elements, performance criteria, range of conditions and foundation skills:

fabricate and assemble one loose and one fitted cover to the seats of two different vehicles.

# Knowledge Evidence

Individuals must be able to demonstrate knowledge of:

work health and safety (WHS) and occupational health and safety (OHS) requirements relating to fabricating and fitting loose and fitted covers to vehicle seats and components, including procedures for:

selecting and using personal protective equipment (PPE)

using specialist tools and equipment

manual handling vehicle components

environmental requirements, including procedures for storing and disposing of waste materials

type, use and basic maintenance of tools and equipment

designs and styles of different types of covers

types of materials and consumables and their applications, including:

fabrics

fasteners

adhesives

methods and techniques for fabricating loose and fitted covers, including:

material selection

planning

measuring

marking out

cutting

sewing

methods and techniques for fitting and fastening loose and fitted covers

procedures for protecting vehicle and components when fitting covers

procedures for final inspection of fabricated and fitted covers.

# Assessment Conditions

Assessors must satisfy NVR/AQTF assessor requirements.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the loose and fitted covers that they have fabricated and fitted, e.g. work orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive or marine trimming workplace or simulated workplace

workplace instructions

PPE required to fabricate and fit loose and fitted covers

materials required to fabricate and fit loose and fitted covers

two different vehicles requiring seat covers to be fitted

tools, equipment and materials appropriate for fabricating and fitting loose and fitted covers.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1