# Assessment Requirements for AURVTW104 Carry out tungsten inert gas welding

Release: 1

Assessment Requirements for AURVTW104 Carry out tungsten inert gas welding

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with AUR Automotive Retail, Service and Repair Training Package Version 6.0 |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

carry out tungsten inert gas (TIG) welding to each of the following material sections:

150 mm long:

aluminium

steel

stainless steel

in the course of the above work, utilise each of the following weld positions:

butt

outside corner

fillet

lap.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

methods to locate and interpret information required to carry out tungsten inert gas welding, including:

manufacturer specifications and operating procedures or equivalent documentation

national standards and codes of practice for welding

workplace procedures required to carry out tungsten inert gas welding, including:

establishing serviceability of tools and equipment

documentation procedures

housekeeping procedures, including:

examination of tools and equipment

storage of equipment

identification, tagging and isolation of faulty equipment

disposal of excess materials

recycling procedures

work health and safety (WHS) requirements relating to carrying out TIG welding, including procedures for:

selecting and using personal protective equipment (PPE) for TIG welding, including approved welding helmets

using specialist welding tools and equipment

using protective screens and fume extraction system

manually handling vehicle components and TIG equipment

identifying and controlling hazards

environmental requirements, including procedures for trapping, storing and disposing of waste materials

different types of shielding gases used in TIG welding

TIG manufacturer operating procedures

material and consumable preparation techniques, including:

aluminium

steel

stainless steel

procedures for setting up and operating TIG equipment, including:

standard and inverter TIG equipment

tungsten electrodes

protective screens and fume extraction system

shielding gas

filler rods

PPE, including approved welding helmets

methods and techniques for TIG welding, including:

welding positions:

butt

outside corner

fillet

lap

weld distortion and distortion control techniques

TIG testing procedures, including procedures for visual or non-destructive weld testing

procedures for identifying and rectifying weld defects, including:

lack of penetration

excessive weld build-up

excess heat

undercut

heat affected zone

procedures for protecting vehicle electronics during TIG welding

procedures for final inspection of welded components.

# Assessment Conditions

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to TIG welding that they have carried out, e.g. work orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

TIG equipment operating instructions

PPE required for TIG welding

protective screens and fume extraction system

TIG welder and shielding gases

TIG consumables

materials specified in the performance evidence requiring TIG welding

tools, equipment and materials appropriate for carrying out TIG welding.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1