# Assessment Requirements for AURVTW109 Carry out basic gas metal arc welding

Release: 1

Assessment Requirements for AURVTW109 Carry out basic gas metal arc welding

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with AUR Automotive Retail, Service and Repair Training Package Version 6.0 |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

carry out gas metal arc welding (GMAW), including:

three butt welds, 150 mm long

three fillet welds, 150 mm long

three steel exhaust pipe butt welds.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

methods to locate and interpret information required to carry out gas metal arc welding on components, including:

manufacturer specifications and procedures or equivalent documentation

Australian Standard 1554

workplace procedures required to carry out gas metal arc welding on components including:

preparing and operating welding equipment

non-destructive testing of completed welds

establishing the serviceability of tools and equipment

documentation procedures

housekeeping procedures, including:

examination of tools and equipment

storage of equipment

identification, tagging and isolation of faulty equipment

disposal of excess materials

recycling procedures

workplace health and safety (WHS) requirements relating to carrying out basic GMAW, including procedures for:

selecting and using personal protective equipment (PPE) when working with GMAW equipment, including approved welding helmet

using specialist welding tools and equipment

using protective screens and fume extraction system

manually handling components and GMAW equipment

identifying and controlling hazards, including:

arc eye and exposure to ultraviolet light

molten metal spatter and fire danger

environmental requirements relating to carrying out basic GMAW, including procedures for:

trapping, storing and disposing of waste materials

GMAW principles, including:

process of using an electric arc to melt and join metal

basic construction and operation of welding gun and wire feed unit

types and application of welding wire and shielding gases

properties and characteristics of metals to be welded

types of welds, including:

butt welds

fillet welds

tee joints

corner joints

procedures for protecting vehicle electronics during welding

GMAW procedures, including:

component cleaning and preparation

welder current and voltage settings, gas flow rates, wire diameters and wire feed speed

wire stick out distance, gun angle between the work pieces and travel angle

weld testing procedures, including non-destructive testing

post-weld inspection procedures, including, weld defects.

# Assessment Conditions

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessment must include direct observation of tasks.

Where assessment of competency includes third-party evidence, individuals must provide evidence that links them to the basic GMAW they have carried out on components, e.g. work orders.

Assessors must verify performance evidence through questioning on skills and knowledge to ensure correct interpretation and application.

The following resources must be made available:

automotive repair workplace or simulated workplace

workplace instructions

PPE required for GMAW, including approved welding helmet

GMAW welder

steel sections requiring GMAW as specified in the performance evidence

tools, equipment and materials appropriate for carrying out basic GMAW.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b4278d82-d487-4070-a8c4-78045ec695b1